Version 1.2 of speedtst.asm. This program probes the maximum conversion

frequency of the DSP-93.

As the conversion frequency increases, there will be occassional "mistakes" in

the output sent to the terminal as the conversion frequency exceeds the

capabilities of the TCL32044.

On my DSP-93, I first see a D/A conversion error at TA=24, TB=13,

freq = 16026. There is no tone produced at that set. Others follow later.

Eventually the program hangs.

The program waits for a character to be typed from the keyboard before

proceeding to allow time to set up a terminal program.

It then loops through values of TA and TB  stored in a table at the end

of the program.

The "inner loop" is less than 60 cycles long which puts a processor

limit of about 160,000 cycles per second which is well above the

D/A conversion limit.

The program generates a tone at each step with the values of TA, TB and

conversion frequency sent to the terminal before each tone.

At some steps, the tone will not be generated. At some point,

the program will hang.

The data "file" at the end of the program has four columns.

TA, TB, calculated conversion frequency, and calculated number of

processor cycles corresponding to this frequency. The table length

is calculated by the program to make it easy to add and delete

table entries. The table included was calculated using an Excel

spreadsheet.

A Timer interrupt routine was added to measure

the time between successive TX/RX interrupts. This measured value

is printed along with the calculated values from the table.

The combinations of TA/TB that produce tones generally have interrupt

intervals larger than "expected" while those that are "silent" have interrupt

intervals smaller than "expected".

Ron Parsons W5RKN