Monitor Memory Usage

12/20/94

The following data memory locations in the current page or absolute

location are used by the monitor.

All the following values are in HEX.

When * is indicated under absolute it means the current AR(ARP) and AR

value are used.

APPLICATION            INDEXED MEMORY          ABSOLUTE        REGISTERS

MEMORY          USED

Reset, Initialization  0,1,2,3,4,5,6,7,8,9     0200 - 020E     AR0,AR1

& Setup + Non callable 60,61,63,64,65,66,67    0400

monitor code.                                  0B00 - 0FFF

1000 - 100E

8018 - 801E

FUNCTION       PAGE      PAGE      CURR.       ABSOLUTE        REGISTERS

0         8        PAGE        MEMORY          USED

--------       ----      ----      ----        --------        ---------

DEBUG          65,66,    6,60,62,               x to 0AFF      AR7

67,68,    7E,7F

69,6A

FUNCTION       PAGE                CURR.       ABSOLUTE        REGISTERS

0                  PAGE        MEMORY          USED

--------       --------------      ----        --------        ---------

GET2HEX        65,66,67,69,6A,     1,2                         none

6B,6D

GET4HEX        65,66,67,69,6A,     1,2,3                       none

6B,6C,6D,6E,6F

GETCHAR        69,6A,6B,6D         1                           none

HEXOUT         69,6A,6B,6D                                     none

HEXIN          69,6A,6D                                        none

INBIT          65,66,6D                        *               current

OUTBIT         65,66,6D                        *               current

OUTBIT2        69,6A,6B,6D                                     none

PRVAL08        65,66,69,6A,6B,6C,6D                            none

PRVAL16        65,66,69,6A,6B,6C,6D                            none

SD_STR         69,6A,6D                        *+              current

SD_CRLF        69,6A,6B,6D                                     none

SP(n)          69,6A,6B,6D                                     none

MWAIT1                                         *+              AR2 & AR3

MWAIT_A                                        *+              AR2 & AR3

PM_BLK_MV      63,64,65,66,67                  *+              current

MP_BLK_MV      63,64,65,66,67                  *+              current

Notes:

Items in [...] are anachronisms and not supported.

ACC is not preserved by the following routines

DEBUG Preserves all register values except AR7 which points to

the stack.  Uses stack memory which starts at location  0AFFh and

moves down less than 0FFh.

GET2HEX gets and echos two  ASCII, converts to hex into ACC and GET2HEXMEM (2h current page)

GET4HEX gets and echos four ASCII, converts to hex into ACC and GET4HEXMEM (3h current page)

GETCHAR gets and echos one  ASCII, converts to hex into ACC and GETCAR_MEM (1h current page)

INBIT   gets data into *

HEXIN   gets data into ACC

HEXOUT  sends data from ACC

OUTBIT2 sends data from ACC [OUTBIT2_MEM (62H page 8)]

OUTBIT  sends data from *

PRVAL08 sends data from ACC [PRVAL_MEM (6H current page)]

PRVAL16 sends data from ACC [PRVAL_MEM (6H current page)]

SD_STR  send zero-terminated string at * [(sends CRLF after string)]

SD_CRLF sends ASCII CR and LF

SP(n)   sends n spaces

MWAIT1  delays 100 msec

MWAIT_A delays (AR2) times 1 msec

PM_BLK_MV  block move

PM_BLK_MV  block move