March 25, 1995 @ 15:50 CST

Hi gang,

Here are the updated versions of the 9600 bps modems which include "live"

DCD and UART reset:

fsk93u1.obj       port 1, std 15% filter (UO-22/terrestrial)

fsk93u2.obj       port 2, std 15% filter (UO-22/terrestrial)

fsk93u1b.obj      port 1, std 18% filter (strong UO-22)

fsk93u2b.obj      port 2, std 18% filter (strong UO-22)

fsk93k1.obj       port 1, ISI compensation filter (KO-23/KO-25)

fsk93k2.obj       port 2, ISI compensation filter (KO-23/KO-25)

LEDs:

1                 not used

2                 not used

3                 PTT

4                 not used

5                 NOT_DCD

6                 DCD

7                 Duty Cycle

8                 Power

LED 5 is in "parallel" with the DCD line to the modem disconnect, and

goes off (low) when DCD is asserted. LED 6 is the inverse of LED 5; it

goes on when DCD is asserted. The DCD action sort of jumps between

these two LEDs. PTT operation on LED 5 was borrowed from W5RKN.

The DSP-93 assembly manual shows the interconnection cable

I am using between the DPS-93 and my MFJ1270C TNC. I recommend that you use

a shielded cable, with the shield tied to pin 15 at each header. Note that

several local jumpers are used on the TNC modem disconnect header. Pin 6 is

connected to pin 10 with a PC trace, shown as a dotted line. Only five

connections are made at modem disconnect on the DSP-93. DO NOT

MAKE ANY CONNECTIONS OTHER THAN THOSE SHOWN.

To set the modem up, tune in KO-23, good and strong, switch the S-meter

to monitor the discriminator and retune to center the meter carefully.

Adjust R215 for about 1 V p-p on pin 26 of the TLC32044.

You should immediately see data,

including the distinctive packet "flag" waveform on pin 17 of the DSP-93's

modem disconnect. I usually trigger my scope from LINE. Ideally, the

waveform will not drift as 9600 is a multiple of 60. In practice the

waveform drifts very slowly.

I tuned my transmitter by first running R214 down to the bottom,

and slowly cranking it up until the transmitter output, monitored on my 2AT,

sounded "half as loud" as un squelched noise. Use a deviation meter if you

are a purist. In the case of my ICOM 275H, it wants about 0.5 V p-p to drive

the transmitter "varactor" input.

All of these modems can be used for both half-duplex and full-duplex

operation.  The TX output from these modems is squelched unless PTT is

asserted. Standard input and output gains have been used (RX gain =

16). As usual, adjust the input signal level at AIO pin 26 to 700 - 900 mV.

This update also adds modem reset via sending an "r" or "R" character to

the modem through the DSP-93 UART.

Notes:

1. For satellite operation and loopback testing, set your TNC to FULLDUP.

2. The TNC DCD light on my UNMODIFIED MFJ1270C does not light with DSP-93

DCD assertion, but the TNC software does correctly hold off the transmitter.

(The DCD light on the MFJ1270C is operated directly by the on board modem for

some reason - thanks to W5RKN for this observation.) I believe Mark Hammond

KC4EBR has developed a mod for the MFJ1270C that operates its DCD light in

sympathy with the DSP-93.

3. DCD is asserted if no input (discriminator noise) is present.

4. Two standard filter bandwidths are offered, 15% and 18%. The 15% filter

generally works best on terrestrial packet and UO-22 signals of medium

strength. The 18% bandwidth filter generally gives better results on strong

UO-22 downlinks. Thanks to Ron W5RKN and Roy W0SL for their work in evaluating

various filters for the FSK modems (this work continues).

5. DO NOT use the modems with the KO-23/KO-25 ISI compensation filter

on UO-22! You won't like the results! Use the modems with the standard

filters. In fact, you may get better results on KO-23/KO-25 at your

station with the standard filters. (I am using a stock ICOM 475H for my

70 cm receiver. The ISI compensation is tuned to this radio.)

6. DCD is a measure of clock synchronization. The amount of jumping

between LED 5 and LED 6 while a signal is being received gives you an

indication of signal quality.

7.  License and Disclaimer

A no cost, non-exclusive license to use the enclosed software is granted

to all users of the TAPR/AMSAT DSP-93 DSP computer, with the restriction

that it be used only for non-commercial amateur radio applications. ALL

other uses are prohibited under this license. Copyright 1991, 1992, 1993

and 1994, 1995 Frank H. Perkins, Jr., WB5IPM. All rights reserved.

This software is EXPERIMENTAL. It is offered AS IS, without Warranty or

Guarantee of any kind. NO representation is made that this software is

suitable for any purpose, or that its operation is reliable. Use this software

strictly at your own risk. The author, AMSAT and TAPR accept no liability

for any loss or injury arising directly or indirectly from its use.

Never use this software under circumstances where its failure to operate

or its failure to operate as expected, or the failure of the attendant

hardware could create a dangerous situation, cause loss of data, or could

cause damage to your amateur radio equipment, computer equipment, or

any other equipment or property.

Please contact Greg Jones at TAPR or the author for information on

commercial licenses of this software.

Frank H. Perkins, Jr.

73, Frank WB5IPM