¹   SHOWTIME 3.00 UPDATE    ¿

(with V3.10, 3.20 and 3.30 notes included)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Dr. Thomas A. Clark

January 7, 1996

Last Update July 4, 1996

SHOWTIME 3.00 is a fairly major update from the earlier 2.xx versions of the "Totally Accurate Clock" ("TAC") support software. Sometime soon I need to make a major documentation update, but these notes, added to the earlier documentation, will have to suffice for now.

The SHOWTIME 3.00 changes include:

- Support for both the older 6-channel and the new 8-channel Motorola ONCORE receivers.

- Inclusion of new debug diagnostics that display the incoming NMEA messages.

- A revision of the Tsigma accuracy predictor algorithm when in position (not timing) mode, and the ability for the user to set the model coefficients.

- A number of changes were made to improve the "smoothness" and reliability of the display updates. The PCrate philosophy was changed.

- The ability for the user to make minor changes in the H/VDOP averaging parameters.

- Menu and *.GPS file changes, mostly minor.

- An attempt (not yet tested) to support some of the advertised Motorola RAIM options (some of which they haven't implemented yet!).

- The ongoing, continuing saga fixing of minor buglets reported to me.

Let me discuss these in order:

(a) 8-CHANNEL SUPPORT: I had already put in most of the necessary hooks for the 8-channel ONCORE receiver, but I finally (thanks to Doug Hogarth of Microsoft & Lyle Johnson of Modular Mining) have had the chance to exercise one of the newer units. The satellite display window can display up to 8 satellites in lock (with SNRs showing) and an additional 4 satellites which are above the ELevation mask limit but which are not being tracked.

(b) DEBUG DISPLAY: In an attempt to sort out some timing problems, I wanted to be able to display the incoming NMEA messages. the [&] key is now a 7-position switch, which increments each time the [&] key is depressed. The [R]efresh Screen key sets the [&] switch back to the no debug (a) position. The 7 switch positions are denoted a,b,c,d,e,f,g and do the following:

- Normal Operation (use [R]efresh to get here immediately).

- Display the incoming $GPZDA NMEA message on the top line. After the ZDA message are displayed X.## and W=##. As before, the X parameter is the time (in steps of 10 msec) showing the internal timer clocking the fraction of the second up to the XTIME time switch. The W parameter is a counter of the number of passes thru the internal loop waiting for data to arrive. At the right-hand end of the top line is a 5-character field like LLbXX where LL is the length of the NMEA message being displayed (normally =36 for the $GPZDA message which has no options), b shows that the [&] switch is in the "b" position, and XX is the time in 10 msec units (just like the X parameter in the middle of the line) when the ZDA message was received. The $GPZDA message format is:

$GPZDA,hhmmss.ss,dd,mm,yyyy,00,00*xx

where  'hhmmss.ss'  is the UTC time (for the last tick),  'dd,mm,yyyy' are the UTC date. The  '00,00'  field is unused by Motorola, but is intended to be  hh,mm  for a local offset time. The  '*'  is the standard NMEA end-of-data flag, followed by the  'xx'  hex check sum, and the message is  terminated with a '<cr><lf>' sequence (not shown and not Included in the LL length displayed at the end of the top line).

c:	The next press of [&] displays the longer (and variable length) $GPGGA  NMEA message. The     	X= and W= parameters are suppressed. At the right-hand end of the line is LLcXX where LL 	and  XX are the NMEA string length and timing parameters as for case b. The $GPGGA 	message format is:

$GPGGA,hhmmss.ss,ddmm.mmm,N,dddmm.mmm,W,n,ss,d.d,hhh.h,M,gg.g,M,tt.t,ii*xx

where 'hhmmss.ss'  is the UTC time of the last position fix (NOTE-- when in zero-D timing mode, this is NOT the current UTC!!). The  'ddmm.mmm' field is the latitude, followed by a  'N/S' north/ south sign. The 'dddmm.mmm'  and  'W/E'  fields are the longitude (SHOWTIME uses a right-handed coordinate system, with W negative and E positive). The single digit  'n'  field is a quality indicator (0=no GPS, 1=normal, 2=DGPS active). The  'ss'  field is the number of satellites currently being used. The  'd.d'  field is the current HDOP. The  'hhh.h,M'  fields are the current height (M=meters), and the  'gg.g,M'  fields are the MSL-to-WGS84 geoid separation in meters [in SHOWTIME.DOC is a long discussion about these parameters. In software versions 5.x and earlier, 'hhh.h' was the height above the WGS84 reference geoid. In later software  'hhh.h' is the height above MSL (mean sea level). SHOWTIME internally uses the WGS84 height, and the geoid switch allows you to interchange the definitions.]. The  'tt.t'  parameter is the age of the last DGPS correction and  'ii' is the 0-1023 serial number for your DGPS beacon if applicable; if there is no DGPS active, these fields have zero length. And like before, the field  '*xx<cr><lf>'  terminates the message.

d:   Like c:, this displays the $GPGSA message and LLdXX at the end of line.

The $GPGSA message format is:

$GPGSA,A,m,ss,ss,ss,ss,ss,ss,ss,ss,,,,,p.p,h.h,v.v*xx

where  'A' (or 'M') denotes Automatic vs. Manual (fixed positions) operation. The single digit  m=1 if a position fix is not available (either because lock is lost, or because the receiver is in zero-D timekeeping mode), or m=2/3 for 2/3-D position fix. The string of 'ss' values (6 or 8 maximum length depending on 6 vs. 8 channel ONCORE receiver) indicate the GPS satellite PRN assigned to each of the 6/8 channels if the satellite is in lock. The total defined length of the 'ss' fields is 12, and unused slots or slots (where the satellite is not in lock) have zero length (i.e. ',,,,'). The  'p.p', 'h.h' and 'v.v' parameters are the current PDOP, HDOP and VDOP values. Again the message is terminated with a  '*xx<cr><lf>'  sequence.

e,f,g: The $GPGSV data arrives in multiple messages. The e: position of the [&] switch gets the first of these. The first digit n of the GSV message is the number of GSV messages to be sent (1,2,3) and the second  is the number of the current message. Each GSV message covers 4 satellites which are in view according to the current ELevation mask. The 3 switch positions (e,f,g) correspond to the  m=1,2,3 GSV messages as defined below [Note: the Garmin receiver uses only 2 GSV messages with 8 satellites maximum]. If n=2 and you use [&] to get to the g (m=3) display, then the f (m=2) message will be "frozen" on the screen. The GSV message  format is:

$GPGSV,n,m,pp,ee,aaa,ss,pp,ee,aaa,ss,pp,ee,aaa,ss,pp,ee,aaa,ss*xx

where  n=1,2,3 is the total number of GSV messages and  m=1,2,3 is the number for this message; only enough messages are sent to accommodate the satellites in view (n=1 for 1-4 satellites, n=2 for 5-8 or n=3 for 9-12).  Each satellite in view has  'pp,ee,aaa,ss' data with 'pp'=PRN, 'ee'=el, 'aaa'=az and 'ss'=SNR in approximate dB [NOTE: The ‘ss’ reported by the Garmin GPS-20 seems to have a ~20 dB bias relative to the Motorola. SHOWTIME automatically subtracts this bias before displaying the SNR.].  If a satellite is in view but is not being tracked, the SNR field is a null (i.e. ',,'). If the full 4 slots in each message are not needed because of the number of satellites in view, the unused  'pp,ee,aaa,ss,'  fields are nulls (i.e. ',,,,'). The GSV messages are each terminated with a  '*xx<cr><lf>'  sequence.

For all the (b-g) modes for which debug is turned on, the decimal point in the ss.sss display now flashes as the 1 second timing loop proceeds, to show that something is happening within the loop. It alternates between the "smiley face" and its inverse until XTIME elapses, and then shows the normal "o" character until the 1PPS tick arrives.

In addition, a new debug feature has been added. When the [_] (underscore) key is hit, a log of all NMEA messages sent by the ONCORE receiver is written. Another tap of the [_] key turns the log off. The log file name is hardcoded to be SHOWTIME.LOG. It was this log file that FINALLY let me trap the bogus and infrequent NMEA message that some recent ONCORE receivers generate (see the reliability section (d) following).

(c) TIMING SIGMA ALGORITHM: As discussed in SHOWTIME.DOC, the basic algorithm for estimating the current accuracy/precision for the TAC/ONCORE receiver is:

sT =  Ö [  a2  +  b2/N ]

where a and b are constants (I suggest a=10 nsec, b=100 nsec for the Motorola ONCORE, or a=b=500 nsec for the Garmin GPS-20),  and N is the equivalent number of degrees of freedom available for averaging out timing errors due to instrumental effects, broadcast ephemerides, tropospheric path delays, ionosphere corrections, multipath, etc. and for averaging the DoD's diddling of the GPS satellite clocks (Selective Availability = SA). In earlier versions of SHOWTIME, we calculated N based on:

N = Nfree = Nsats - Nmin

where Nmin is the minimum number of satellites needed for the current mode:

Nmin = 1 for zero-D timekeeping (not available in the Garmin GPS-20),

or   	Nmin = 3 for 2-D positioning,

or   	Nmin = 4 for 3-D positioning,

and  Nsats = the number of satellites being tracked and Nfree is the number of "extra" degrees of freedom available.

When running in 2-D or 3-D positioning mode, the Tsigma algorithm used in earlier versions of SHOWTIME is overly optimistic about the Tsigma being achieved. While Nfree = the number of degrees of freedom is correct, not all this "strength" is used to improve timing; a part of the Nfree "strength" is applied to improving the positional accuracy.

It is beyond the scope of the real-time SHOWTIME calculations to figure the detailed covariance matrix necessary to apportion the Nfree degrees of freedom amongst the Nmin parameters being determined, so we might adopt an algorithm which equally apportions the degrees of freedom like:

N = Nfree/Nmin   =    (Nsats - Nmin) / Nmin

but this algorithm yields N=0 for Nfree=0 (i.e. Nsats=Nmin), which will cause the Tsigma algorithm to blow up with a "divide by zero" error; clearly this is to "harsh" since the timing solution is not degenerate when N=0. Hence in SHOWTIME 3.00 I have adopted the "softer" non-degenerate algorithm:

N = [Nsats - (Nmin - x)] / Nmin

where x=.5 seems to fit the measured parameters fairly well, yielding estimated 3-D Tsigma values of (283,164,127,107) nsec for Nsats=(4,5,6,7) with the Motorola ONCORE.

So that the user can adapt SHOWTIME to local measuring conditions, it is now possible to change the a and b Tsigma error model coefficients in the line in your SHOWTIME.GPS (or *.GPS) file:

MSigma       10   100                	| Tsigma Model for the Motorola ONCORE

or 	Msigma     500   500                   	| Tsigma Model for the Garmin GPS-20

In this case the default ONCORE values are shown which have proven to be about right for 1PPS averaging times of a few seconds; you must edit the *.GPS file to put in Garmin values. If you run with longer measurement averaging times, you may want to adjust the second (b) coefficient to a lower value.

(d) PCRATE, RELIABILITY and "SMOOTHNESS" CHANGES: A number of changes were made in SHOWTIME 3.00 to improve the "smoothness" and reliability of the display updates.

A major philosophy change involves the PCrate parameter. If your receiver is sending all 7 possible NMEA messages (ZDA, GGA, GSA and 3 GSV messages), the total time spent sending messages can be in the 800-900 msec range. For the proper update of the time display, the ZDA message is the most critical (we cannot use the time in the GGA message since it is invalid when the receiver is running in the Zero-D timekeeping mode). Motorola allows the various NMEA messages at different rates, and the PCrate parameter is included in SHOWTIME to try to prevent "overload" when the full 7 messages are being sent.

Unfortunately, Motorola has hard-coded two annoying properties into the NMEA messages:

- The ZDA message is always the LAST message sent during each second even though it is the most important message for timekeeping.

- There seems to be no way to reliably assign a message to a particular second "slot" when the messages are sent at a rate slower than once per second. I have tried many slot assignment schemes, and I haven't found a reliable method (it was my futile attempts to try to invent a slot assignment algorithm that led to my inclusion of the NMEA debug display options described above.)

In terms of the 4 types of NMEA messages, we note that the different messages have different priorities:

- The ZDA message is most important since it conveys the correct time, and SHOWTIME is really a clock display program for the “Totally Accurate Clock”.

- The GGA message is important when determining position, since it conveys the Lat/Lon/Hgt data. It is also important because it reports the number of satellites being tracked. It only conveys HDOP.

- The GSA message is less important, but it conveys the PDOP, HDOP and VDOP and the number of satellites being tracked can be inferred. It also reports on 0/2/3-D position operation.

- The GSV messages are the least important since they give only the satellite az/el/SNR data.

With this in mind, the PCrate algorithms have been changed thusly:

- PCrate = 0 delivers all NMEA messages at a one/sec rate.

- PCrate > 1 yields variable rates according to the priorities in this table:

PCrate    ZDA    GGA    GSA    GSV

1        1s     1s     1s     1s

2        1s     1s     2s     4s

3        1s     1s     3s     6s

4        1s     2s     4s     8s

with PCrate now has having values 1 thru 4 (formerly the limit was 3).  [Note: For the Garmin receiver, PCrate is always = 1]

If you are having timing problems that can't be corrected with PCrate > 1, the problems are probably occurring when 3 GSV messages are being sent because there are more than 8 satellites above the elevation limit. You can improve this case by raising the elevation mask so that fewer satellites are being reported. At mid-latitudes, there are seldom more than 8 satellites above 10 degrees.

Other reliability/smoothness changes include:

- The position averaging scheme has been changed. Formerly the position average was only updated when a new set of positions arrived with a GGA message. Now the last position received is averaged each second. This change made for smoother operation and made more sense with the addition of HDOP/VDOP weighted averaging added in SHOWTIME (ver. 2.85).

- A lot of work was done to improve the reliability, stimulated by a new bug found in Doug Hogarth's 8-channel ONCORE. Occasionally, but always at a time xx:00:24 (xx:00:23 before the Jan.1 Leap Second) a cryptic and hard to trace "divide by zero" error appeared. This particular time matches the time that the GPS satellites can do an ephemeris update. It turned out (caught by the new [_] data logging option) that the ONCORE was sending bogus NMEA messages at the magic time. The UTC hh:mm:ss repeated the previous second's time, and the HDOP/VDOP/PDOP parameters in the GSA and GGA messages were all 0.0, which in turn was causing problems with the weighted position calculations. Since this error only happened once or twice per day, it was a BITCH to find!

- To trap bogus NMEA data in earlier versions of SHOWTIME, I had used a lot of arithmetic tests for data validity. All these have been stripped out and replaced by a NMEA string validation test based on the “official” NMEA hex check sum: the modulo 127 XOR of all the ASCII characters between (but not including) the "$" and "*". If the check sum fails the NMEA string is ignored. The ONLY arithmetic validity test surviving is the one found necessary to eradicate the bogus DOP/VDOP/PDOP bug just described.

(e) USER CHANGES IN THE H/VDOP AVERAGING PARAMETERS: I added the ability to weight the position averages with HDOP (Lat/Lon) and VDOP (Hgt) in SHOWTIME 2.85. The exponential averaging algorithm (described in more detail in the old SHOWTIME.DOC file ) is still used:

New Avg = [  W*(new position) + (Length-W)*(old Avg)  ] / Length

The Length parameter defines the averaging window; it starts at a value of 1 and increments by 1 each second up to a "clamp" window length (set in the [A]veraging screen or the *.GPS file). Until Length reaches the clamp, we get uniform averaging, and after it hits the clamp the averaging becomes exponentially weighted with a time constant related to the clamp.

Before v2.85, the weight W was defined as 1. In 2.85 and later releases, the weight W can also be defined by the relevant xDOP parameter (x=H for Lat/Lon or x=V for Hgt) like

W = [xDOPnominal / xDOP]2

In v2.85 I hard-coded HDOPnominal=1.5 and VDOPnominal=2.0 as typical "good" values which would normally result with weights W close to 1.0; this allows the averaging window "time constants" to be similar for the weighted and unweighted cases. In SHOWTIME 3.00 I let you pick the nominal values in the *.GPS file in a line which reads like:

DOPWt    1   1.50   2.00               |Weight Posn Avg

The initial digit (1 or 0) turns xDOP weighting switch on/off (also available in the [A]veraging screen with the [D]OP Weight toggle). The next two numbers are the nominal "good" HDOP and VDOP parameters. You may want to revise these parameters for the 8-channel ONCORE (1.20 and 1.70 seem to work fairly well).

(f) MENU AND *.GPS FILE CHANGES: The main changes you will note on the screen are in the lower left-hand corner "positions" window. Some errors were discovered and hopefully fixed in the logic driving the top line in the  when satellite tracking was marginal. The window now updates just after the second, reflecting all information gathered during the previous second. The Tsigma now/max/min/avg line has been changed for esthetic reasons.

Occasionally when you make a [G]PS mode change from 0-D timekeeping to 2-D or 3-D positioning, the positions display may not "pop up" instantly. If this happens, just hit <esc>. Sometimes after making mode changes, the HH:MM time field may be blank. Just hit the [R]efresh Screen key.

The old switches [/] and [\] that allowed the position and satellite windows to be stopped have been deleted. I needed the code space for all the other features and nobody reported that the feature was useful.

The [=] timing summary in the upper right-hand corner was revised for esthetics reasons and to (hopefully) make it more self-explanatory.

There are a few minor changes in the *.GPS file format with SHOWTIME 3.00, but your old files should still work OK. The SATOFF and POSOFF lines have been deleted, and new DOPWT and MSIGMA lines (discussed above) have been added. Running the new SHOWTIME 3.00 code with an old *.GPS file, and then doing a [F]ile [U]pdate will reformat the old file for you. The current *.GPS file looks like:

******** (replaced by a later version - see v3.30 notes on pages 10 & 11)

(g) RAIM ADDITIONS: Motorola has partially implemented RAIM integrity testing in the latest ONCORE firmware. According to their documentation, a future RAIM augmentation will allow the 1PPS pulse (and hence the front panel LED) only be present when satellites are in lock. I added the switches to enable this option per their documentation, but it is untested (and undelivered from Motorola, at least as of the Jan.95 date on the documentation). In particular, the *.GPS file (see the sample *.GPS file above) and the [T]iming screen now has a RAIM entry followed by an integer ## parameter. If ## = 0 then the RAIM testing is turned off. If the ## is in the range 3 to 32767, then than value is the 100s of nsec error that is tolerated before the RAIM algorithm triggers. Because Motorola has not fully documented their RAIM responses, I have no idea what will happen if the RAIM alarm triggers!

(h) BUGLETS and GLITCHES: A number of small tweaks (too mentionable to be numerous) have been made in an attempt to make the software be more reliable.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Ver 3.10  Updates -- April 1, 1996

Corrected a leap-year error that made 1996 dates after Feb.29 be bad. Try as we may to test all the possible cases, we never seem to catch all the problems!

Added 4th digit to decimal portion of last Lat/Lon since some versions of the receivers now report an extra digit.

Restructured the NMEA parsing routines to make them more sanitary.

Added minimal support for the Garmin GPS-20 "TAC Lite". To use a Garmin, set ApType=5 in your *.GPS file.

In anticipation of some future changes, the structure of the Rcvr: line in the lower-left hand quadrant has been revised.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

SHOWTIME v3.20 Updates:

Version 3.2 contains several incremental enhancements to the earlier versions of SHOWTIME, hopefully all in the direction of improving things!

(1) Improved Error Handling: When running on slow PCs or in multitasking environments (especially Windoze 95), the RS232 serial input routine can have a buffer overflow and cause the program to die. I see this at my home when things get flaky during automatic time scheduled maintenance periods (SCANDISK, Disk Compression, etc.) in the wee small hours of the morning.

I think I have found a better "fix" than trying to rely on Quick Basic's error handling and MAYBE 3.20 will improve things. I did the testing here by trying to get SHOWTIME to crash in W95 with 5 copies of "dir /s" plus SCANDISK running on top of SHOWTIME.

You will notice two minor changes associated with this fix. With the [&] debug running in position #1 (with ZDA messages) a 3rd counter shows a "sticky" version of the count of the number of characters in the RS232 input buffer. A few counts are not unusual. When it gets up to ~100, the computer is running behind. When it reaches 400 (representing about one second of unread data in the input buffer), the "fix" flushes the input buffer and resets the RS232 communications. Also, when the RS232 input is beginning to get "constipated", the decimal point in the time display

changes to a "smiley face" as a warning.

(2) More W95 compatibility: When running in window mode, W95's DOS "box" allows you to use either a default character set or "True Type" characters. Unfortunately, several of the TT characters were incompatible with the character set I used in SHOWTIME. A few minor changes (like deleting Greek letters and making minor changes in the line drawing characters used)

now make the display be legible with either type font.

(3) GEOID Corrections Revisited: In a much earlier version, I added a 2-position switch (GEOID) to accommodate two different Motorola Geoid conventions. Early PVT6 receivers sent WGS84 heights in their NMEA GGA message. But the NMEA "rules" specify that Mean Sea Level (MSL) heights should be reported and Motorola changed their ONCORE firmware. Unfortunately

they got the sign of the Geoid height correction wrong, and the newest ONCORE receivers are (supposed to be -- I haven't seen any yet) changed to fix that error. So there are THREE kinds of Motorola receivers out there, and the Geoid Switch has been changed to allow for all three receivers breeds (and for the Garmin GPS-20's which appear to be correct like the

latest Motorola units). I went ahead and made the switch have 4 positions as shown in this table, with positions 0 & 1 preserving the definitions used in earlier versions of SHOWTIME, position 2 used for the new receivers and 3 to allow for some really pathological future case:

GEOID SWITCH      |  MSL or WGS Height  |  Geoid Sign  |  Symbol

---------------------+---------------------+--------------+----------

0 (antique MOT PVT6) |      WGS            |     OK       |   "."

---------------------+---------------------+--------------+----------

1 (old MOT ONCORE)   |      MSL            |    Wrong     |   "-"

---------------------+---------------------+--------------+----------

2 (new MOT+Garmin)   |      MSL            |     OK       |   "+"

---------------------+---------------------+--------------+----------

3 (pathological case)|      WGS            |    Wrong     |   "x"

---------------------+---------------------+--------------+----------

The column headed "Symbol" shows the single character symbol I chose to use to display the Geoid correction in the Rcvr Status line. In the [G]PS Mode Screen, the [W]GS switch toggles thru these possibilities and the value is saved as GEOID in the *.GPS file.

(4) Height Displays: After I cleaned up the Geoid Sign problem in (2), I added another feature that had been requested by several people – the ability to work interchangeably in WGS84 or MSL heights. The GEOID switch you find in the [G]PS Mode screen actually has 8 positions. The first 4

are as described above and correspond to the old SHOWTIME convention with heights converted from MSL (as reported by the receiver) to WGS84 Geoid heights (using the correction reported by the receiver, after the sign error is fixed) in accordance with conventional geodetic practice. (See Ver 3.30 notes for more on the GEOID saga.)

When SHOWTIME is running, a new [W]GS/MSL switch appears on the help menu. This switch serves to toggle the height display function and can be used at any time. The height convention selected by either the [G]PS [W]GS switch or the real-time [W]GS/MSL toggle is saved when you write a *.GPS file.

Internally, SHOWTIME <<ALWAYS>> runs in WGS84 coordinates and toggling between WGS And MSL heights only effects the display and does nothing to any position averaging that is occurring.

(5) Improved GARMIN Support: My "GARMINizing" of SHOWTIME is not yet complete. A Garmin (or similar) receiver lacks much of the configuration flexibility of the Motorola ONCOREs. You cannot operate in a Zero-D timing mode, or set elevation limits, or turn the Ionosphere corrections on/off, etc. My plans to accommodate the simpler receiver have begun. The

main thing you will see is that much of the [G]PS Mode screen disappears when you select the Garmin=5 application type.

When ApType=5 is selected for the Garmin (or similar) receiver, the source of Date/Time information changes from the $GPZDA message to $GPRMC since most of the simpler receivers lack ZDA support. Note that the RMC date format is packed as a single DDMMYY value, while the ZDA format is like DD,MM,YYYY.

(6) Also fixed a bug that crept in that prevented offsetting the 1PPS timing epoch. Don't know when this happened but it's OK now.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

SHOWTIME 3.30 Update Notes:

The main changes between SHOWTIME 3.22 and 3.30 concern receiver specific items:

- MOTOROLA: After finally sorting out the GEOID sign situation, I got tired of manually having to edit the *.GPS file to accommodate the different corrections, so I added an automatic detection feature. During the INIT@9600 phase, the receiver is queried to fetch the configuration. It looks for a 5.x or 6.x receiver and sets GEOID=0, or GEOID=1 for a 7.0 thru 8.6 receiver release or GEOID=2 for receivers 8.7 and later (which I still haven't seen). The user should no longer need to worry about this problem! The GEOID switch has been removed from the [G]PS mode screen.

After the INIT@9600 message, the receiver info is displayed on the screen briefly. If you want to get make note (like doing a screen print), hit any key after you see the INIT@4800 message and the program will commit suicide and stop after displaying the information.

The order of the lines in the *.GPS file format has changed slightly, and several lines have been reformatted. An old file should work fine.

In doing all these changes, I stripped out the code that kept the configuration checksum. After trying it, I decided it really wasn't very useful.

- GARMIN: SHOWTIME now supports the limited list of GARMIN receiver options. You can manually select between 2D and automatic 3D operation. If you plug in a non-ONCORE receiver, the software will automatically select the proper APTYPE and GEOID switches.

The *.GPS file needed for the GARMIN is smaller than for the Motorola, so when you do a [F]ile [U]pdate the un-needed information is stripped out. The timing sigma information now displays an estimate of the timing accuracy even for the GARMIN. I haven't "calibrated" it well enough to develop timing model parameters, and the software defaults to Motorola parameters. You can change these in your *.GPS file in the MSIGMA line. An initial guess would say that the GARMIN case is something like

MSIGMA  500 500

i.e. an instrumental "noise" of about 500 nsec and sqrt(n) term of comparable size (the  canonical Motorola vales are 10 and 100 nsec respectively).

- BOTH MOTOROLA and GARMIN: SHOWTIME has limited ability to automatically detect MOTOROLA vs. GARMIN receivers, automatically setting the APTYPE parameter.

The detailed structure of the *.GPS files has changed, but old files should work OK. After getting the receiver to run, it is recommended that you do a [F]ile [U]pdate to re-write the new format *.GPS file. The *.GPS file for the GARMIN has fewer entries since many parameters are not applicable.

For the Motorola receiver, a typical *.GPS file now looks like:

-------------------------------------------------------------------------

####  < SHOWTIME v3.30 >  File   22 June 1996 =  #174  17:42:55.000

#### Posns:  Ref             Avg            Delta    RMS     #Sec

LAT:     +39.1883333365   +39.1883877673    +6.048  14.81    2748

LON:     -76.9349999746   -76.9350733235    -6.317  10.82    2748

HGT:         +108.000          +95.814     -12.186  41.38    2631

REFID  Testing SHOWTIME v3.30

GEOID    0      -34.00                 |

#### Control                 ----------+----------

COM1                                   |COM1/2

TZ:      UTC  0                        |Time Zones: PC

LTZ:     EDT  4                        | Local

TimeSwap UTC                           | Initial

AUTOSET  240                           |AutoSet rate

XTIME    .9                            |Time Seq Slice

LOCKHYST 2                             |Alarm Hyst,s

PCRATE   1                             |PC Update,1-3s

#### Avg Windows & Statistics----------+----------

DOPWt    1   1.50   2.00               |Weight Posn Avg

MSigma  10  100                        |Tsigma Model

POS     10000                          |Pos

RMS     10000                          |RMS

SigmaT  10000                          |SigmaT

Tsigma  283  127  149.0995  2810       |max/min/avg Tsigma,#Sec

Locked  #174  16:54:31.000 1           |Since,#Unlock

#### Receiver Modes          ----------+----------

APtype  4                              |Ap:Fixd

NDim    3                              |0/2/3-D

ELEV     10                            |Min El

AIV                                    |All-in-View

PDOP                                   |xDOP Selection

Select by xDOP                         |not Highest-in-Sky

IONO     1                             |Ion+

RAIM     5                             |RAIM 500ns

IGNORE  +++++++++++++++++++-+++++++++++|Accept=+

; PRN=31^       21^       11^        1^| PRN31=>1

Exit     Binary                        |@9600

#### Timing Info             ----------+----------

Epoch      +0.0000000000               |Epoch Offset,sec

Cable     -92.400                      |Cable,ns

Inst       -8.000                      |Rcvr Delay,ns

Early      +0.000                      |Early offset,ns

#### Ticks                   ----------+----------

Stone    1000                          |Sec tick,Hz

Mtone    1200                          |Min tone,Hz

SLong    0                             |Min tone@sec

#### Colors in order:        ----------+----------

##  BkGnd,BigUTC,Calndr,SatInfo,Epoch,Faint,Frame

##  Msec Offset,Posn,DOP/Stat,Help,Red Warning

Colors: 1  14  10  10  12  9  8  7  5  13  14  12

#### END OF FILE

-------------------------------------------------------------------------

and for a GARMIN GPS-20, a *.GPS file looks like:

------------------------------------------------------------------------

#### < SHOWTIME v3.30 >  File   22 June 1996 =  #174  03:06:10.000

#### Posns:  Ref             Avg            Delta    RMS     #Sec

LAT:     +39.1883333365   +39.1882037753   -14.396  15.27      31

LON:     -76.9350000000   -76.9353430818   -29.546  28.87      31

HGT:         +108.000         +108.000      +0.000   0.00       0

REFID  Testing SHOWTIME v3.30

#### Control                 ----------+----------

COM1                                   |COM1/2

TZ:      UTC  0                        |Time Zones: PC

LTZ:     EDT  4                        | Local

TimeSwap EDT                           | Initial

AUTOSET  240                           |AutoSet rate

XTIME    .8                            |Time Seq Slice

LOCKHYST 5                             |Alarm Hyst,s

PCRATE   1                             |PC Update,1-3s

GEOID    2      -34.00                 |

#### Avg Windows & Statistics----------+----------

DOPWt    1   1.50   2.00               |Weight Posn Avg

MSigma  500  500                       |Tsigma Model

POS     10000                          |Pos

RMS     10000                          |RMS

SigmaT  10000                          |SigmaT

Tsigma  742  742  741.6199  31         |max/min/avg Tsigma,#Sec

Locked  #173  21:27:21.000 0           |Since,#Unlock

#### Receiver Modes          ----------+----------

APtype  5                              |=   GARMIN

NDim    3                              |0/2/3-D

#### Ticks                   ----------+----------

Stone    1000                          |Sec tick,Hz

Mtone    1200                          |Min tone,Hz

SLong    0                             |Min tone@sec

#### Colors in order:        ----------+----------

##  BkGnd,BigUTC,Calndr,SatInfo,Epoch,Faint,Frame

##  Msec Offset,Posn,DOP/Stat,Help,Red Warning

Colors: 1  14  10  10  12  9  8  7  5  13  14  12

#### END OF FILE

---------------------------------------------------------------------

~~~~~~~~SHOWTIME REVISION HISTORY ~~~~~~~~~~

Ver 1.x  -- Developmental "Beta" releases.            [Dec'94 to Apr'95]

Ver 2.0 --- The first "public" releases of SHOWTIME.  [April 1, 1995]

Ver 2.10 -- May 23, 1995

- Added the alarm unlock indicator feature.

- Minor bug fixes & display enhancements.

Ver 2.20 -- June 1, 1995

- Improved the unlock alarm functioning from 2.10.

- Added [L] toggle switch to turn the unlock alarm on/off.

- Fixed the "clock sticks in zero-D" bug (due to "feature" in Motorola's $GPGGA message that doesn't report time in Zero-D).

- Added the [W]GS84 GEOID correction on/off switch ([P]os & [G]PS screens and GEOID in the *.GPS file) to accommodate Motorola's new (Version 8.x) firmware changes in the way heights are reported.

- Added  [D]ate/Time Coldstart Clock initialization in [P]os screen.

- Stripped out a lot of spurious code and cleaned up a lot of the screens to make them more legible and to save code space.

- Improved wording on [SPACE BAR] main help screen menu.

- Deleted the [#] Channel number display (which nobody was using).

- Several menu inputs changed from manual "type a number" entry to "rotary switches" -- for these, you press the key to cycle thru the legal values ( [%]APtype, [S]elect DOPtype, [I]on, [W]GS Geoid,  [$]PCrate and [N]oise ticks all run this way now).

- Satellite Status "billboard" rearranged to make room for the new 8-channel ONCORE display (Untested - I haven't tried an 8-Ball yet!).

Ver 2.21 -- June 2, 1995

- Corrected a minor bug in reported heights when running in positioning mode with old (pre-V8.0) ONCORE code. The [W]GS84 switch in the [P]osition screen now reads "new" vs. "old" to correspond with pre- and post- Version 8.0 receiver firmware.

- Entering a [M]anual position in the [P]osition screen now "seeds" the average position with the new reference position.

- In the [M]anual [H]eight entry, you can now enter either WGS84 or MSL heights and SHOWTIME will do the conversion for you,  providing the [W]GS GEOID switch is in the correct position for your receiver's firmware. The correction is based on the WGS84 minus MSL Geoid separation computed in the Motorola receiver (the value of which is reported to you on the [P]osition screen).

Ver 2.22 -- June 4, 1995

- Cleaned up & added some more logging info to the *.GPS file. The logging data in the file now includes the number of seconds included in the Lat/Lon/Hgt/Tsigma averages, the "Valid Since" last locked time, and the number of times it was unlocked. Note that these parameters reflect the last time you write the *.GPS file with a [F]ile [U]pdate.

- The [P]osition screen now shows the RMS of (REF minus AVG)

- When you are in zero-D timing mode, the reported Average position is set to the 3-D REFerence position. In 2-D mode, the Hgt is set to the REFerence Hgt.

- Removed the [F]ile [K]ill *.GP$ command (not needed with [S]hell)

Ver 2.23 -- June 19, 1995

- Made a few minor changes to make software recover from temporary data outage on RS232 cable better.

- Small change to screen that tells when PC clock has been [S]et.

Ver 2.30 -- August 1, 1995

- The Lock Alarm logic that was added in Ver 2.10 had problems! A new, more robust logic was added in 2.30. The problem occurred because of the way that I counted the number of satellites that were in lock using the GSA message. The new logic uses the GGA report directly from the receiver. SHOWTIME now has an added parameter (LOCKHYST in the *.GPS file, which can be set manually on the [G]PS Mode Screen) to act as a hysteresis "damper" to suppress brief dropouts. A value ÷ 2-3 seconds for the hysteresis seems to work well.

Ver 2.40 -- Sept. 11, 1995

- Corrected minor bug in entering cable lengths -- Meters units didn't work correctly, and the units code got confused if the units had an "S" in them (i.e. MSEC).

- The Time Offset screen was made a bit more explanatory. Several other cosmetic screen changes also were made.

- The Timing Offset "mini-screen" in the upper right-hand corner (toggled on/off with the [=] key) was streamlined and (hopefully) made more explanatory.

- The (ham) Maidenhead Grid Square computation was streamlined.

- Change to RS232 COM1/2 port initialization seems to improve glitch recovery reliability.

Ver 2.50 -- Sept. 16, 1995

- Added the ability to allow the user to disable & ignore multiple GPS satellites. In the [G]PS Screen, the [G]PS Use/Ignore key allows you to toggle any of the satellites PRN01-31. (You cannot toggle PRN32 on/off. This is because the ON/OFF status is sent to the Motorola as a 32-bit unsigned array, one bit/satellite. Alas, QB45 does not have an unsigned long integer type, and it would have been a huge pain to add the 32nd bit to the field. Sorry!). This was added because the antique Block-1 PRN12 satellite has been unreliable. In the [G]PS modes screen, good satellites are shown as "+" and bad ones are flagged "-". The "-" character also is shown next to ignored satellite's PRN in the satellite status window in the lower right-hand quadrant of the screen.

- A new parameter "IGNORE" appears in the *.GPS control file to save the status of the new parameter as a 31-character string where a "+" means that the satellite is to be used an any other character (like a "-") means that the PRN is to be ignored.

- To make room for the Ignore feature, the ability to exit SHOWTIME with any of the 7 "legal" NMEA messages being set to user-defined rates ("EXIT NMEA") has been discarded. "EXIT BINARY" still works.

- In general, a lot of cleanup was done on various screens to make the SHOWTIME code more compact. Also, several small, related subroutines were merged and/or put into inline code to make the program a bit smaller and faster.

- The display that shows the testing of the 1PPS RS232 tick at startup has been changed from ....#..#.... to something that looks more like an oscilloscope trace.

- The calculation of Local/Greenwich Mean Sidereal Time (LMST/GMST) was streamlined. Earlier versions included the quadratic (but not cubic) term to calculate the GMST at 00:00 UTC based on elapsed time from the J2000 epoch. For computational efficiency, the small quadratic term is now "locked" to 1996.0. The error for the next several years is < 1 msec.

Ver 2.51 -- Sept. 27, 1995

- In 2.40 & 2.50 I streamlined the RS232 configuration. Unfortunately, it didn't work on all machines. Some RS232 ports default to having the CTS and RTS lines low and some default to high. The 2.51 software has been modified to IGNORE the unused handshaking lines. (thanks to the folks at Onsala for giving me an opportunity to duplicate this problem).

Ver 2.60 -- Oct. 15, 1995

- A lot of work was expended to better trap RS232 I/O problems. The PC's 16450/16550 UART is now issued a hard reset each time the I/O is initialized. Error trapping has been improved. Several variables are now tested to make sure they don't go beyond reasonable bounds if the PVT-6 receiver send some garbage data.

- The DCDmask variable was hard-coded instead of being set in the *.GPS file, so users MUST input the 1PPS signal on the DCD line.

Ver 2.61 -- Oct. 17, 1995 (minor maintenance fixes)

- Still had to trap a couple of more bad data instances. Minor update from Ver. 2.60

Ver 2.70 -- Oct. 22, 1995

- Did a lot of testing to force errors that could cause SHOWTIME to die and implemented a LOT of new traps. All these involve possible bogus data returned in NMEA messages if the computer loses sync with the RS232 NMEA messages while running in W95.

- To make space for all the new traps, a number of separate subroutines were made into in-line code or subroutines in the MAIN module. This should also speed execution a bit.

- A few minor cosmetic changes were made to the display,

Ver 2.71 -- Oct. 24, 1995 (minor maintenance fixes)

- Found another bad data sanity test that was needed.

- Cleaned up and slightly revised the [F]iles screen logic

Ver 2.72 -- Nov. 4, 1995 (never released publicly)

- Fixed minor bug in the display of the nsec portion of the [U]+[I] total time offset in the [T]iming screen and the [=] display window. Calculations were done correctly -- the display was wrong because an integer number was displayed in a floating point field.

- The PCrate switch somehow had gotten broken a few versions ago.  It was made to operate again and the logic to set the GGA/GSA/GSV NMEA messages at a slower rate (PCrate seconds) was revamped to be more reliable at both 2 & 3 second "slow" rates. The new logic may glitch if you try to start SHOWTIME within about 10 seconds of the PC's Midnight marker.

Ver 2.80 -- Nov. 6, 1995

- Made a number of changes in the logic behind the Position/Timing window so that the top line more correctly reports the receiver's status and that the unlock detection is more reliable.

- Additional "sanity" checks were added to prevent bogus coordinates from being included in the position (especially heights).

- Cleaned up & tested the new widgets added in 2.72 for public release.

Ver 2.81 -- Nov. 8, 1995

- A minor bug caused an error in [F]ile [L]oad -- a new *.GPS file was not loaded correctly. A global variable got used for two different  purposes during one of the recent changes.

Ver 2.82 -- Nov. 11, 1995 (minor maintenance fixes)

- The earlier versions of SHOWTIME would report incorrect Lat/Lon if either Lat or Lon was between 00 and -01 degrees. This is a common problem with a lot of software since it is hard to represent a signed negative zero integer!  This minor problem is now fixed.

- The -00 problem was not trivial to fix. Added a new parsing function  (Pd$) to make it possible. This resulted in a slight display change, namely that Longitudes under 100 degrees (+ or -) now have the  degrees field zero-padded to 3 characters plus sign.

- I also made it possible to [P]osition [M]anual enter Lat/Lon in the  -00 to -01 degree band.

- Fixed a minor bug that somehow crept in that made the [C]olor and [A]veraging screens not work right when in zero-D timing mode.

- The [&] "speedometer" has a second number displayed -- this is the number of times thru the inner loop trying to read an NMEA message  during the current second.

- A screen which shows the "Locked Since" and # Unlocked information appears when a non-operative menu key is struck. <esc> clears this information panel and resumes operation. (The same info also appears when you manually [S]et the PC's clock.)

Ver 2.83 -- Nov. 14, 1995 (minor maintenance fixes)

- Found a minor glitch (which crept in from earlier changes) when running in forced 2-D mode with SHOWTIME not recognizing the reported rcvr state properly.

- Fixed glitch where [A]veraging [z]ero didn't reset all the reported statistical parameters - some variables had been left out of COMMON.

Ver 2.84 -- Nov 21, 1995 (minor maintenance fixes)

- Fixed minor bug that caused program to lock up if you tried to  [F]ile [L]oad a non-existent file -- an IF test was wrong.

- It was possible for the [P]osition or [R]MS or [T]iming Sigma filter lengths to not "clamp" at the proper values if you decreased the numbers and they were already greater than the new filter lengths.

- The Pos/RMS filter weights had been a mish-mash of floating and integer values. They are now uniformly long integers and a single subroutine handles incrementing them each time new data arrives.

- There had been some confusion on the Position screen about dAvg & dRef. The nomenclature was changed to d(RA) (meaning the difference Ref-Avg), d(AL) (meaning Avg-Last) and d(RL) (meaning Ref-Last). [NOTE: This was later changed again to (R-A), (A-L) and (R-L)]

- Some essentially useless (and probably buggy) code that attempted to sort the "S-meter" display panel when two satellites are at the same elevation to have the rising satellites listed above the setting satellite was deleted. The ordering for this rare case is now only determined by the order the satellites are reported in the NMEA msg.

Ver 2.85 -- Dec 1,1995

- In response to a request, the 2-D or 3-D positions can now be averaged based on (1/HDOP)2  [Lat/Lon]  or  (1/VDOP)2  [Hgt].  A new switch [A]veraging [D]OP Weight has been added -- pressing the [D] key will toggle the xDOP weighting on/off. In the normal Position window, you can see when the weighting is turned on by a letter "w" appearing before the positions differences and RMS indicators.

- A new switch has been added to the *.GPS file called DOPWEIGHT.  If the line is present, then the xDOP weighting is to be applied, if it is absent, then the weighting is not used.

- To squeeze the [D]OP Weight item into the [A]veraging screen, the [A]veraging screen layout was changed. To accommodate this change, the weighting filters inside SHOWTIME were changed from LONG (integer) to DOUBLE (floating). The filter length strategy is similar to that used before. See FAQ #23 for more details.

- The original “SHOWTIME.DOC” file documents the software to this point.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Ver 3.00 -- Jan.7, 1996   -- A major update release.   SHOWTM30.DOC documents the changes

from this point forward. Changes in 3.00 include the following:

- Support for both the older 6-channel and the new 8-channel Motorola  ONCORE rcvrs.

- Inclusion of new debug diagnostics that display and permit logging of the incoming NMEA messages.

- A revision of the Tsigma accuracy predictor algorithm when in position (not timing) mode, and the ability for the user to set the model coefficients.

- A number of changes were made to improve the "smoothness" and reliability of the display updates.

- The PCrate philosophy was changed.

- The ability for the user to make minor changes in the H/VDOP averaging parameters.

- Menu and *.GPS file changes, mostly minor.

- An attempt (not yet tested) to support some of the Motorola RAIM options (some of which they haven't implemented yet!).

- A more graceful way to allow for a complete receiver coldstart was  included.

- And the ongoing, continuing saga fixing of minor buglets reported  to me.

Ver 3.01 -- Jan. 13, 1996 (minor maintenance update)

- Fixed a minor bug in the help menu triggered by the [space bar]. The help menu now also displays the [_] NMEA message log key.

Ver 3.02 -- Jan. 16, 1996 (minor change)

- The reference hex checksum added in v3.00 didn't include all the parameters uploaded to the receiver in the Init@9600 binary load. This was changed and the checksum now is 'touched' by each and every bit sent to the receiver. The line in the [F]iles screen that reports the checksum was reworded slightly. (Note – checksum deleted in Ver. 3.30)

- Back at Ver 2.60 I had added a hard reset for the PC's 8250/16450/16550 UART to try to improve error recovery. This caused problems  on a COMPAQ (and perhaps other machines) so the hard reset was  removed.

- The coding that generates the square boxes around the different  frames was streamlined, saving nearly 200 bytes of code space.

Ver 3.10 -- April 1, 1996

- Corrected a leap-year error that made dates after Feb.29 be bad.

- Added 4th digit to decimal portion of last Lat/Lon since some  versions of the receiver firmware now report an extra digit.

- Restructured the NMEA parsing routines to make them more sanitary

- Added minimal support for the Garmin GPS-20 "TAC Lite". To use a Garmin, set ApType=5 in your *.GPS file.

- In anticipation of some future changes, the structure of the Rcvr: line has been revised.

Ver 3.20 -- May 28, 1996

- Hopefully we have made another incremental reliability improvement with a new "fix" to preclude against RS232 input buffer overruns.

- Expanded the GEOID switch to accommodate new (8.8) ONCORE rcvrs.

- Added option so user can use [W] to toggle between MSL and WGS84 height display.

- Added some more Garmin GPS-20 support (not yet complete).

- Fixed a bug that crept in sometime (who knows when) that lost the ability to offset the msec portion of the 1PPS time epoch.

Ver 3.21 -- June 1, 1996

- Two of the revised GEOID switch settings were inadvertently reversed in 3.20, making for a minor incompatibility with earlier versions of SHOWTIME.

Ver 3.22 -- June 17, 1996

- I still didn't have the signs right for the GEOID correction. Hopefully now they are correct!

- Changed the order that some information is posted to the screen. Now the position screen is updated immediately on receiving the  $GPGGA message.

Ver 3.30 -- June 23, 1996

- MOTOROLA: SHOWTIME now interrogates the GPS receiver to get the software version and sets the GEOID switch automatically. The receiver configuration information is displayed after INIT@9600. Hitting any key after INIT@4800 will stop SHOWTIME so you can read/save the configuration info.

- GARMIN: All the GPS-20 options now should work. When running with a GARMIN receiver, many of the menu screens are considerably abbreviated, deleting entries which are not applicable.

- SHOWTIME has limited ability to automatically detect MOTOROLA vs. GARMIN receivers, automatically setting the APTYPE parameter.

- The detailed structure of the *.GPS files has changed, but old files should work OK. After getting the receiver to run, it is recommended that you do a [F]ile [U]pdate to re-write the new format *.GPS file. The *.GPS file for the GARMIN has fewer entries since many parameters are not applicable.

- The binary configuration checksum generated from the commands uploaded to the Motorola receiver has been deleted.

Ver 3.31 -- July 4, 1996 (minor changes)

- Minor formatting changes to line up the HGT+WGS above the displayed height.

- Deleted DCD time-out at 4800 baud.

- Small display change in GARMIN mode.

- Apparently the Motorola Geoid sign change applies for firmware  >= 8.8, so the automatic detection algorithm was changed accordingly.

- [G]PS mode screen streamlined a bit.

- The GPS week is defined as [0000-1023] and "rolls over" in August, 1999. The displayed GPS week (computed from the MJD in SHOWTIME)  now rolls over properly.

- Improved reliability of logic to get MOTOROLA firmware revision automatically.