TOTALLY ACCURATE CLOCK

HARDWARE.DOC

_______________

Tom Clark

June 5, 1995

This set of notes is intended to document my implementation of the TOTALLY

ACCURATE CLOCK ("TAC") hardware. Detail is included for those who are using the

TAC boxes we supply as well as those who intend to fabricate virtual clones.

--------------------------------------------------

(1) The Motorola GPS Receiver "Engine" in the TAC:

The TAC is based on the Motorola OEM GPS receiver module called the "ONCORE

BASIC" (also sometimes denoted by the model name "PVT-6"). To deliver high-

accuracy timing, this receiver MUST be equipped with "Option A, 1PPS Output",

although other options (carrier phase, generate differential corrections, etc.)

can also be enabled. The minimum configuration carries the Motorola part number

A11121P116 or A11121P117 and (as of March, 1995) costs $535 each in the USA (sans

antenna and documentation). Motorola has also sells full "designer kit" of the

receiver called the "CORE EVAL" model (including antenna and full documentation

and with all possible firmware options enabled) for about $1100 USA.

The basic Motorola receiver board is 70x100 mm in size (mounting holes spaced

60x90 mm) with a 10-pin power & I/O "insulation displacement" connector (IDC)

and a series OSX (also called MCX) series RF input connector on one end. The

10-pin IDC connector has the following pin-out [Note: Motorola uses non-standard

pin numbers in their documentation and this sketch follows their designations]:

top  PVT6  bottom

5  10

+9 to 36 VDC----o  o---------------o RS232 Gnd

o  o------<--------o RS232 TXD to GPS

Power Gnd----o  o------>--------o RS232 RXD from GPS

+5 VDC-------o  o------>--------o 1 PPS Gnd

+ Battery----o  o------>--------o 1 PPS output

1  6

Notes:

(a) Use either the +5V  OR  the +9-36V power input BUT NOT BOTH!! Therefore

only pin #2 or #5 should be connected. In the TAC we use +12 VDC connected

to Pin #5 and pin #2 is floating.

(b) Pin #4 is used by Motorola as +Vpp for re-programming the ONCORE receiver's

Flash EPROM and should not be used.

(c) Motorola jumpers the 3 ground pins (#3, #7 and #10) together on the

receiver board. In the TAC, pins #3 and #7 are jumpered to cabinet ground

and pin #10 is carried to the RS232 I/O connector.

This particular Motorola receiver was chosen for the TAC project because it has

repeatedly been shown to have � 50 nsec accuracy and precision, and because it

was readily available! A separate document by Mike King et al entitled "TEST

RESULTS AND ANALYSIS OF A LOW COST GPS RECEIVER FOR TIME TRANSFER APPLICATIONS"

[presented at Inst. of Navigation meeting, Jan.1994] documents the receiver

performance. With Mike's kind permission, a reprint of this paper is included

with the TAC documentation; I have appended at the end of the King et al paper

some of my own comments on the receiver performance.

Motorola makes several other related ONCORE receiver products. The "ONCORE VP"

is a smaller, less expensive version of the ONCORE BASIC. The VP receiver lacks

the on-board DC/DC power converter of the BASIC and requires +5VDC power; hence

pin #5 of the 10-pin IDC connector described above is not connected. The

computer I/O in the VP version has logic levels of 0/+5V (not RS232) but uses

the same pins. The 10-pin IDC connector and the OSX RF connector are on

opposite ends of the circuit board. And the VP circuit board is smaller than

that used in the BASIC version. Motorola also markets a version of the BASIC

with the receiver mounted in a nice metal box. I have had neither of the VP or

boxed versions for testing -- all my experience (and the TAC project) are based

on the ONCORE BASIC version.

Motorola has recently announced an 8-channel version of the ONCORE. I eagerly

await having one to test to see if the additional 2 channels improve the

performance of the BASIC unit used in the TAC project.

--------------------------------------------------

(2) Augmenting the Motorola Receiver:

To make this receiver suitable for use in the field, I decided that a few

features were needed in addition to the ONCORE BASIC board; thus was borne the

TAC project. Key to this is an add-on circuit board (the same size as the basic

receiver) which includes the following functions:

(a) The primary purpose for the TAC is to provide a high-accuracy 1 PPS signal

to the user. The "raw" receiver pulse is buffered thru high-speed (74AC04)

logic to provide fast rise-time, low-impedance drive to the user.

(b) We have also found it desirable to provide time synchronization for an

attached computer. The 1 PPS signal is converted to RS232 logic levels and

sent to the computer on the DCD handshaking line.

(c) The RF performance of the Motorola receiver is improved by adding a small

MMIC L-band amplifier. This also allows us to easily change the DC bias

voltage sent to the antenna-mounted preamplifier from the +5VDC provided by

the Motorola receiver.

(d) I have been doing some experimenting with RTCM SC-104 DGPS (differential

GPS) corrections. The Motorola receiver has only one RS232 port, so the

differential corrections must be "OR-ed" with data coming from the computer

to the receiver.

(e) I have found it useful to have a visual "pilot light" indication that the

receiver is working. The add-on board has up to three open-collector

outputs for such purposes. These outputs can also be used as drivers for

negative-going 1 PPS signal outputs.

(f) Motorola makes provisions for a battery to keep a coarse time-of-day clock

running and some BBRAM (Battery Backup RAM) to make the receiver cold-start

more rapidly. The add-on board charges a NiCd battery to provide this

function.

(g) I always worry (because I have done it myself!) that I will make some

stupid mistake that burns out an expensive piece of hardware! The add-on

board provides "stupidity fuse" functions to help minimize this

possibility. If you do something stupid on the 1 PPS output, the worst you

can do is to zap a 74AC04 logic gate. If you short the antenna, you may

burn out an RF choke or a 7805 regulator chip. If you goof on the RS232

I/O, diodes and current limiting resistors protect the receiver. If your

antenna is hit by lightning, the MMIC should blow up before the receiver

does. And if you reverse power polarity, a diode and fuse will protect the

receiver!

In my definition (and what we are supplying to our "customers") a complete

"TAC" also includes the following:

(h) A "lab instrument" cabinet for the receiver: For this, I found a nice,

commercial "made-in-Taiwan" RS232 A/B switch box which we strip down and

repaint. We use the existing 9-pin connector holes in the switch box for

computer and RTCM SC-104 I/O. I also "invented" a very neat way to make

professional looking labels for the panels (documented in Appendix B).

(i) A power supply suitable to run the instrument: We selected a commercial

"brick" switching supply normally used for a lap-top computer to provide

+12VDC. This supply in "international" and can run from 100-250 VAC at

either 50 or 60Hz [You must supply the standard "IEC" AC power cable to

meet your local needs].

(j) A small GPS antenna: The TAC's we are supplying include one of two

different brands of antennas (Motorola or Trimble). Both require +5VDC

"bias" (provided by the TAC box) on the antenna cable to power the

preamplifiers. Both have magnets that will hold them on ferrous surfaces.

The Motorola antenna is gray "mushroom" , 10 cm in diameter and 4 cm high;

it is equipped with a short "stub" of RG58 coax and a BNC connector. The

Trimble antenna is a white "hockey puck", 6.5 cm diameter and 4 cm high; it

is equipped with a short stub of small RG174 coax cable and a BNC

connector.

I would suggest that either type be used with a ground-plane at least one

wavelength (20 cm) in diameter. The Motorola magnet is quite strong, but

for a permanent installation, I would suggest a dab of RTV silicone rubber

adhesive be used to glue it down. The Trimble antenna has 4 each (3.7 cm

square pattern) 4-40 tapped holes for permanent installation.

Both antennas have built-in low-noise L-band preamplifiers with at least 30

dB of gain to make up for cable losses and can drive at least 20 meters of

RG58 ("� inch") coax. If you have a longer cable run, you will probably

want to use RG8/RG214 ("� inch") series coax.

(k) Software: Two sets of IBM-PC based controller software are available for

the TAC. One set is provided by Motorola; they provide the "official"

documentation only with the "CORE EVAL" designer kit version of the ONCORE,

but I am providing you with my own versions of the essential documentation.

The Motorola software allows you to exercise ALL the receiver functions.

The second software "widget" is my personal "SHOWTIME" code, which turns

your PC into a real clock and allows (what I believe to be) a more "user

friendly" interface to the critical receiver functions. We are considering

adding some "TAC" support functions to the Mark-IV VLBI "Field System"

control software if there is a sufficient user demand for it.

--------------------------------------------------

(3) Hardware Implementation -- Overview:

Let me now discuss the specific implementation of the TAC. BNC connectors on the

front and rear panels have separate buffered 1 PPS signals. The 1 PPS signal

from each connector at zero volts for 800 msec and rises to +5V for 200 msec,

with the leading edge coincident with the UTC second (default). The leading edge

can be offset from -999.999 �sec (early) to +999999.999 �sec (late) from UTC in

1 nsec steps under software control (see the SHOWTIME documentation for

details). A LED indicator on the front panel flashes synchronously with the 200

msec output pulse.

On the rear panel is another BNC connector to connect to the GPS antenna. A BNC

series connector may seem to be sub-optimum for use at L-band (1575.42 MHz), but

it has proven adequate. (The inner portion of a BNC connector is virtually

identical to N-series commonly used at such frequencies.)

Also on the rear panel are two DA-9 female connectors for RS232 signals. One is

used to connect to a computer and it follows the typical IBM-PC/AT 9-pin RS232

connector pinout:

TAC RS232                  PC RS232   -or-    PC RS232

(9-pin)                    (9 pin)           (25 pin)

DATA gnd      5-------------------------5-----------------7

TXD to GPS    3-------------------------3-----------------2

RXD from GPS  2-------------------------2-----------------3

DCD input     1-------------------------1-----------------8

The RS232 communications with the computer are either at 4800 or 9600 baud. The

Motorola receiver automatically selects 9600 baud when operating with the

Motorola-supplied controller software, when all communications are in Motorola's

proprietary binary format. When using my SHOWTIME software, both 4800 and 9600

baud speeds are used. Control data is sent from the PC to the receiver at 9600

baud using Motorola's proprietary control format to set up receiver modes, and

communications revert to 4800 baud NMEA-format ASCII transmissions during normal

SHOWTIME operations. Pin 5 is normally the Carrier Detect (DCD) status, and the

RS232 DCD signal is low (� -6v) for 800 msec, rising high (� +6v) for 200 msec

coincident with the main 1 PPS signal. The other pins on the connector are

unused.

The second DA-9 RS232 connector is used for optional RTCM SC-104 DGPS

corrections. The RTCM corrections MUST be sent to the receiver at the same 4800/

9600 baud speed used for communications to/from the receiver. The RTCM SC-104

signals are simply ORed with the PC signals, and they may collide. The use of

RTCM should be considered experimental and is not an "official" TAC function!

On this connector, Pin 1 is RS232 ground, and the RTCM SC-104 signals may be on

either pin 2 or 3. A 3-position switch selects (pin 2) vs. (pin 3) vs. (OFF).

The final connector on the rear panel is a standard coaxial power connector of

the type used on laptop computers, transistor radios, etc.; the small switching

power supply we provide with the TACs is fully compatible with this connector.

This coaxial power connector uses a 2.1 mm diameter center pin and the center

connector is "hot" at +12V nominal (voltages from � +9V to � +28 V should work

OK). If you inadvertently connect the wrong polarity or connect to an AC source,

a shunt diode inside the TAC will protect the hardware and blow a small (1 amp,

slo-blow metric-sized) fuse in the receiver.

Inside the box, the Motorola receiver board and the add-on board are mounted on

threaded bushings on an aluminum plate inside the box, with the receiver mounted

below. Also mounted on the plate are a small 1 Amp "slow-blow" fuse and a 9V

NiCd battery. The receiver and the add-on board are interconnected by a short

10-pin cable and a piece of semi-rigid coaxial cable.

_______________

(3a) Circuit Details of the TAC Add-On board:

We now discuss the details of the add-on circuit board (PCB) mounted above the

Motorola receiver inside the box. The PCB size is 70x100 mm with corner holes at

60x90 mm to match the Motorola receiver board. A flat 10-pin IDC cable provides

all signal/power connections between the two boards, and a semi-rigid coax cable

provides the RF connection. The overall PCB parts layout is shown in Figure 1.

The board has several functions which are discussed separately. In figure 1, the

MMIC RF amplifier occupies the central portion. A regulator to supply +5VDC

power, LED driver transistors and a 74AC04 high-speed CMOS buffer are on the

left. The upper right-hand corner contains connections for DC bias for the

antenna-mounted preamplifier, including provisions for an optional regulator for

non-standard voltages. Below the optional regulator is a DC/DC converter (using

a 555 timing oscillator) to make � -9VDC for the RS232 I/O. The lower right-hand

quadrant contains a dual op-amp used as an RS232 output buffer, RS232 I/O

connections, and the 10-pin connector header to interface with the Motorola

receiver board via the flat ICD cable. We will now discuss the detailed

implementation of these sections separately.

_______________

(3b) RF amplifier and antenna bias circuitry:

In the center of the TAC add-on board is a single-stage L-band amplifier using a

Monolithic Microwave Integrated Circuit (MMIC); the schematic of this section is

shown in Figure 2. The MMIC amplifier "pill" is a Mini-Circuits MAR-6 that costs

< $2.00. The MAR-6 is supplied +5V power by the ONCORE receiver thru the coax

cable. Per Mini-Circuits design "recipe", a 110 ohm resistor provides the MAR-6

with the proper operating current. Since this amplifier operates at L-band,

FIGURE 1 GOES HERE

FIGURE 2 GOES HERE

FIGURE 3 GOES HERE

parts layout is important. Figure 3 is a sketch with more detail on parts

placement. More details on the coax cable connecting the RF amplifier on the

add-on board and the ONCORE receiver are contained in Appendix A.

This RF amplifier also provides the DC bias necessary for the antenna-mounted

preamplifier. A push-on jumper allows you to remove the bias. Note that the

"production" TACs we provide do not make use of the optional bias circuitry

since the antennas we include require +5VDC bias. +12V DC bias can be supplied

by simply changing a jumper. Other bias voltages could be obtained by using the

optional U5 bias regulator chip as described in Figures 2, 3 and 6.

_______________

(3c) RS232 and 1 PPS circuitry:

Figure 4 is a schematic of the RS232 I/O circuitry. A dual "741-type" op-amp

(typically TLC272 or LM1558) is used as an RS232 driver. U2A simply buffers the

ONCORE RS232 output with a unity-gain amplifier to act as a "stupidity fuse".

U2B acts as a CMOS-to-RS232 level converter and inverter to generate a 1 PPS

RS232 signal. Both stages have 470 ohm series resistors for protection and

stability. A diode OR-gate allows both the computer and an RTCM SC-104 DGPS

signal to be passed to the ONCORE receiver. These diodes and a diode clamp to

the +12V supply rail provide some "stupidity" protection for the receiver in

case the RS232 interface voltages go berserk.

Figure 4 also shows the simple NiCd battery charger. The 3k resistor limits the

charging current to � 1 ma. A fully-charged battery will provide backup for the

ONCORE board for about a month -- the main function for this battery is to keep

a small "wrist-watch" clock on the receiver running when power is disconnected.

The receiver stores GPS almanac data in Flash EEPROM memory, but it needs to

know the date and an approximate time to know which GPS satellites are visible.

If the battery is discharged, initial GPS acquisition times will be long since

the receiver will have to search the entire GPS constellation to get date/time

information.

Since the battery charging current is so low, it takes over a week or two for a

fully discharged battery to be fully charged. If this causes difficulty,

increase the charging current by changing the resistor R9. Putting a 470 ohm

resistor across R9=3kohm will increase the charging current to about 10 ma.

Figure 4 also shows the interconnection between the TAC add-on board and the

ONCORE receiver and the RS232 I/O connectors on the rear of the cabinet.

Figure 5 is the schematic for the 1 PPS output buffers. The 1 PPS user output

signals are buffered thru 2 stages of 74AC04 logic gates. 74AC series logic was

chosen as the fastest CMOS logic family. The propagation delay thru two stages

is � 8 nsec. The 74AC04 is a hex inverter. One stage (U1A) is used as an

inverter and the other 5 stages are available as output drivers; multiple

sections are paralleled to lower the output impedance. Three isolated user

outputs are provided -- U1B/C and U1D/E with 2 paralleled stages and U1F as a

single stage. All 3 have provisions for independent reflection-snubbing

terminations (denoted Rout on Figure 5). In the distribution TACs, the three

Rout output resistors are all 22�. The U1D/E and U1F outputs are connected in

parallel to the front panel, while U1B/C are connected to the rear panel output.

Figure 5 also shows three optional open-collector outputs, which are normally

intended for driving LEDs. The board has space (not used) for a LED to be

mounted on the TAC add-on board. In the distribution TACs, two open-collector

inverters are provided, but only one is used to drive the front-panel LED

indicator thru a 510 ohm resistor. This output can be used as a low-impedance 1

PPS output which will go low at the UTC epoch. Jumpers (not used) exist on the

board so that the polarity of any of the open-collector outputs can be inverted.

_______________

(3d) Power Supply Circuitry:

Figure 6 shows the on-board power supply circuitry. U4 is a conventional 7805-

FIGURE 4 GOES HERE

FIGURE 5 GOES HERE

FIGURE 6 GOES HERE

type regulator which supplies +5VDC for on-board logic and +5V antenna bias.

U3 and its associated circuitry is a small DC/DC converter based on a 555-series

(we use a CMOS TCM555 or 555CN chip) oscillator running at � 2.5 kHz (frequency

determined by the RT1/RT2/CT network). The output of the 555 is a square-wave,

which feeds a voltage doubler. When driven by +12VDC (and when loaded by the

rest of the circuitry) this circuit produces � -8VDC (despite the bus being

labeled -12V on the drawings). A 12V zener diode provides a clamp in case an

input supply voltage higher than +12V is used. This circuit is NOT elegant, but

it works and makes an adequate RS232 output signal.

Also in Figure 6 note that a 1 AMP 1N4001 diode is placed across the input to

the board. The sole purpose for this diode is to blow the 1A fuse in the

unlikely case that the wrong input polarity (or an AC signal) is applied to the

power connector on the TAC. The consumer industry seems to have no standard on

the polarity used on small coax power plugs. Some wall-mounted power supplies

have the center pin negative. Some common wall-mounted supplies are merely

transformers that provide AC. The fuse and diode are a cheap insurance policy!

If the fuse should happen to blow, the fuses we use in the production TACs are

miniature metric style fuses (very common in Europe). In the USA, Radio Shack

does have replacements available. On one occasion, I had a 1A "fast-blow" fuse

burn out when a unit was connected. Apparently the charging of some electrolytic

capacitors caused a big enough surge to blow the fuse. I recommend the use of a

1A "slow-blow" fuse to avoid this problem.

Figure 6 repeats some of the information in figures 2 & 3 concerning the use of

the optional U5 regulator to provide non-standard antenna bias voltages.

--------------------------------------------------

(4) Some other information:

The original TAC hardware prototype was done as a "home basement" project during

the 1993/4 winter, at which time an initial "hand etched" version of the add-on

board was designed. That design was cleaned up for the professional TAC

"production". As of this writing, various groups have signed up for nearly 50

copies of my design! My group at NASA/Goddard, working with our VLBI/Hydrogen

Maser support contractor AlliedSignal Technical Services Corp. (ATSC), are

endeavoring to make the units available to the community at our cost (about

$1100 per copy).

If parts of this design/documentation are incomplete (or seem a bit crude or

unprofessional), I offer the defense that all the circuit board design,

schematics, software, etc. have all been produced in my basement at home using

the personally-owned design tools I had available. If you need schematics, they

are done in ORCAD. Circuit board layout was with rather antique Wintek software.

Various of the support drawing were done with AUTOSKETCH or TURBOCAD -- in

short, a real hodge-podge!

In building the TAC we "invented" a few construction tricks that might be of

interest to others. Appendix A describes the scheme we invented to make

professional looking front panels using CAD software and a LaserJet printer.

Appendix B describes the "invention" of a $0.02 OSX connector and how we

fabricate the semi-rigid coax cable that interconnects the add-on board to the

ONCORE receiver board.

--------------------------------------------------

(5) The TAC and the Internet:

We have established an Internet mail exploder called gps-timing for the purpose

of coordinating "TAC" users and communicating low-cost GPS-based time/frequency

ideas amongst several disciplines (VLBI, SLR, Computer Networking, Amateur

Radio, etc.). Mail to the group can be posted to

gps-timing@aleph.gsfc.nasa.gov

You can subscribe to gps-timing by sending an Email message to Nancy Vandenberg

nrv@bootes.gsfc.nasa.gov

I am making my aleph host available as an anonymous FTP file server

for TAC and related materials. The host name is

aleph.gsfc.nasa.gov

Begin in the ~/GPS subdirectory on aleph to find whatever goodies are there. I

am placing TAC-related documentation and software in the directory

~/GPS/totally.accurate.clock

_______________

Acknowledgements:

I want to offer special thanks to Irv Diegel and Chuck Kodak of ATSC for their

help in "productionizing" the TAC. Bernt Ronnang at Onsala got me started on

this project while I was on sabbatical in Sweden in the summer of 1993. Mike

King of Motorola provided invaluable assistance in making available "beta"

firmware, controller software and documentation after he found out that I had

independently discovered a timing bias problem in the early PVT-6's.

Alan Rogers of Haystack deserves credit (or blame) for pushing me into making

copies of the TAC design available to the VLBI community. John Bosworth and John

Degnan at NASA/GSFC have pushed me into making the design available to the SLR

community. Nancy Vandenberg has helped by handling the administration of the

gps-timing Email exploder.

Lyle Johnson gets credit for making PVT-6 ONCORE receivers available to some

crazy people in the amateur radio community who wanted high accuracy time/

frequency available for experimentation.

Special thanks to my wife, Elizabeth for showing patience above and beyond the

call of duty as I disappeared into the basement night after night to work on

the TAC project!

To all these people, my thanks for your help and support!

Tom Clark                                                         June 5, 1995

Email:

clark@tomcat.gsfc.nasa.gov

tac@aleph.gsfc.nasa.gov

w3iwi@amsat.org

APPENDIX A

Low-cost OSX-series coax connectors

and semi-rigid coax used in the TAC

The Motorola receiver uses a special coax connector, series OSX (also called

MCX) for its RF input. While these connectors are very inexpensive in large

quantity, we were unable to find any source for small quantities at costs under

about $10 each. The "TAC" began as a "home basement" project and I found this

situation unacceptable, so I decided to invent my own OSX connector. This note

describes my "invention" for posterity.

In the SMA connector series used with 0.141" semi-rigid cable, male connectors

consist only of the threaded shell. The center conductor of the coax cable is

used as the center pin for the connector. I observed that the center conductor

of the small 0.085 semi-rigid coax cable was nearly the same diameter (slightly

larger) as the center pin in the "official" OSX series connector. The shell of a

male OSX connector snaps into the mating female connector and has ~0.130" outer                                                                  -

diameter.

Thus was borne the idea -- can we find/fabricate a suitable connector shell and

use 0.085 semi-rigid cable. I came up with two solutions to this problem, both

of which involve only about $0.02 parts cost.

Solution #1 is what we are using on the production TACs. We found that the

cylindrical sleeve on a (particular size of) ordinary crimp solder-lug (the

portion you crimp onto the wire to make the connection) had just the right outer

diameter to mate with the chassis-mounted OSX connector on the ONCORE. After

showing him how the cylindrical portion of these lugs could be made to work

after, Irv Diegel kludged a fabrication jig to keep everything lined up. The

only problem is that we could not find a lug with an inner diameter of 0.085" so

that the lug's sleeve can be slipped over the coax (like in an SMA connector).

When assembled with the jig, they work fine! The only caution is to make certain

that the center "pin" is properly centered in the shell after soldering --

otherwise they can mangle the teflon insert in the receiver's connector!

I will now document solution #2 in more detail. This solution involves the use

of 0.125" tubing obtained at a local model-airplane hobby store. The preparation

procedure is as follows:

(A) Straighten a 6" piece of 0.085"

semi-rigid coax cable. Use a knife

to score a notch in the cable

shield about 1 cm from the end. The

easiest way is to roll the tubing

on a flat surface with the knife

blade as shown. Hold the short end                      [SKETCH A]

in a pair of pliers and gently

bend. The cable shield will snap on

the line you scored with the knife.

Carefully (do not nick the wire!)

cut partly thru the Teflon dielec-

tric. Twist the end to shear the

Teflon and remove the 1 cm end.

(B) Tin 5-10 mm of the shield. This

will cause some Teflon to extrude,

so you may need to re-trim the                         [SKETCH B]

Teflon so that �1 mm is exposed.

(D) Now we have to prepare the shell. Take a length of 0.125 inch "hobby brass"

tubing. Stick the end into the receiver's RF connector and you will find that it

fits very loosely. Use a tapered piece of metal (I use a center punch) and flare

the end of the tubing slightly. Check the fit again. Iterate until the brass

tubing fits snugly into the connector. If you push it in all the way, it will

"click" into a groove in the connector. You are trying to get a snug fit BEFORE

it clicks into place.

With a fine-bladed saw (or a tubing

cutter), cut off a 6 mm (�") piece

of the tubing. Slip this sleeve

over the prepared end of the coax                   [SKETCH C]

and solder, leaving about 1 mm of

the coax conductor exposed. Center

the center conductor in the shield.

For the not-so-faint of heart, I have had better luck in getting everything

centered and aligned by using an OSX female connector as a jig while doing the

soldering step. For this, I have used a real $500+ ONCORE receiver as this

soldering jig! If you want to risk it, you insert the prepared coax into the

receiver connector. Then slid on the sleeve and push it into the receiver

connector shell. Then solder. I haven't blown a receiver yet!

(D) Now, having a cable with the home-made connector,  we prepare the coax to

properly mate the add-on board to the receiver. Assemble the two boards with the

standoffs you plan to use. Insert the coax cable thru the large hole in the add-

on board into the receiver input connector. On the cable, mark the place where

the cable comes thru the top board:

[SKETCH D]

(E) Remove the cable and bend into a "shepherd's crook" shape with your fingers.

Check the fit, and adjust until the cable fits like in this sketch:

[SKETCH E]

Trim the cable to the proper length so that the center conductor will match the

microstrip line on the MAR-6 amplifier section. On the cable, mark the end of the

long ground area on the PCB. Remove the coax and trim the shield to this mark.

Re-assemble and solder the shield and center conductor. The PCB has some holes

suitable for "horseshoe" shaped loops of wire to help hold everything together.

(G) When you make the final assembly and power up the receiver, the end of the

coax should have +5V on it coming from the ONCORE receiver board. If +5 is not

present then the cable is shorted, probably because the center pin in the

"pseudo-OSX" connector is bent! You should also see about +3.4V on the output

lead of the MAR-6 MMIC amplifier ("down-stream" from the 110 ohm series resistor).

If anyone finds a cheap source for "real" OSX connectors, please let me know!

APPENDIX B

SOME COMMENTS ON THE "TAC" CABINET

AND A CHEAP WAY TO MAKE PROFESSIONAL

LOOKING PANEL LETTERING.

The Box:

The box is a "made in Taiwan" commercial RS232 "A/B" switch box which we stripped.

The rear panel has three pre-punched holes for 9-pin RS232 connectors.The TAC

uses two of these holes and our "serial number" label covers the third. The front

panel had hole where the switch was mounted already suitable for a BNC connector.

Associated with the switch was a small hole to prevent rotation which we used

for the front-panel LED. After drilling the rear-panel holes, we spray painted

the cabinet with "Hewlett-Packard Gray" paint to cover up the old lettering.

This approach of using a commercial RS232 switch box resulted in a professional,

attractive metal box for a much lower cost than using custom-made sheet metal!

The receiver/add-on board stack, battery and fuse are mounted on a sheet of

aluminum, which in turn is attached to the "roof" of the box. Rather than

drilling unsightly holes in the "roof", the metal panel is glued to the box with

silicone RTV adhesive.

The 9-pin A/B switch boxes are Model # C4794, which cost $36 each from:

Global Computer Supplies

1050 Northbrook Parkway, Dept. 62

Suwanee, GA  30174

The Labels:

Experience has shown that lettering of panels for electronic equipment is

normally a costly, time-consuming job if the unit is to look nice. A lot of

widgets I have built have been labeled with stick-on paper labels, felt-tip

marker pens, etcetera simply because I didn't want to go thru the hassle of

preparing silk-screens or sending the hardware off to have the panels engraved.

With the TAC, we have "invented" a new way to do this job cheaply and quickly. I

therefore vow to never again use the paper label/felt-tip pen approach!

The scheme involves the use of computer-generated labels and a LaserJet printer.

The panel layout is done with a CAD program and a copy is printed onto sticky-

backed paper. This paper layout is used to drill the panels, and then you peel

the paper off and discard. At this point, you de-burr, clean (and re-paint, if

necessary) the panel.

You now print another copy of the panel drawing on the LaserJet, this time using

a translucent frosty adhesive mylar sold by drafting supply stores and called

"Repro Film". It is normally used by draftsmen for putting standard title

blocks on mechanical drawings.

The adhesive material is carefully applied to the panel, working from the center

outwards. Try to avoid air bubbles, but if they happen, prick the film with a

pin to allow the entrapped air to escape. Finally trim the outer edges to the

size of the panel.

As you can see with the "production" TAC's, the Repro Film works well on smooth

painted surfaces. It also looks very nice on bare aluminum, providing you brush

the aluminum with steel wool first to give it a "silky" appearance. The

LaserJet lettering on Repro Film is very rugged and durable. I defy you to

scratch the lettering off with your fingernails!

The Repro Film is made by Rayven, Inc. and is Rayven's type # 310. We obtained

it as catalog item # 08-RAY-310 ($40 for a 100 sheet box) from:

The Re-Print Corporation

P.O. Box 830677

Birmingham, AL  35283

######################################################################################