TrakBox_Turbo version installation info.

========================================

Dallas semiconductor released 80C32 code compatible high speed CPU

DS80C320.  This chip has upper compatible with Intel 8032/8052 series

CPU and three times faster with the same crystal.  There are many other

additional functoins (two serial ports, watchdog timer, many interrupt

sources, high X-tal clock etc..), and now, TrakBox can work with this

CPU.   The calclation speedup is 4 times faster than ordinary T_B.

You need (1) DS80C320 CPU,  (2) 22.118400MHz X-tal (3) 320TRK EPROM

[Install]

1.At first, you must certificate your TrakBox is working with original

EPROM and CPU. (8031,32,51,52 CMOS or NMOS).

2.Off the battery back up jumper (JP13)

3.Replace the original CPU with DS80C320.

Check the T_B is running with new CPU.  The speed up  1.5 or 2 times

faster than the original.

4.Replace the EPROM with Turbo EPROM (320_xxx.ROM).

Check the T_B can run with COM port baud rate 4800.

5.replace the original X-tal(11.0592MHz) with 22.1184MHz.

6.Now, Your TrakBox_Turbo is completed.

[Notice]

1) New CPU with 22Mhz clock rate needs high access time EPROM

and RAM. In my case, I used 120ns EPROM (Mitsubishi M5M27C512K-12)

and 80ns RAM (62256LP-8).  If your T_B doesn't run, you may replace

with high speed EPROM(-12 means 120nSec access time).

Many beta tester reported that turbo code run with fast EPROM

and RAM from many supplyer.

2) Dallas Semiconductor released preliminaly DS80C320 chip and

the early lot  has bugs in micro code.  The present version

has xxxxA6-yyy on the lavel.  If you have xxxxA3-yyy version, this

chip is not usable for TrakBox Turbo.

[Dallas Semiconductor info.]

DS80C320	$12.00

DALLAS SEMICONDUCTOR

4401 South Beltwood Parkway

Dallas,  Texas  75244-3292

tel: (214)450-0448

Fax: (214)450-3715

overseas call: (214)450-5351

Credit card sales service: 1-800-336-6933

[Parts source information by VE2ALQ]

-----------------

For those people out here looking for components

to upgrade TrakBox to turbo following was (is) my source for every

component needed:

JDR Electronics

Phone 408-494-1400

FAX   408-494-1420

North America Toll Free  Phone 1-800-538-5000

Fax   1-800-538-5005

Good luck folks this is where I got all of mine, no problem with

single GTY orders.  Have a Good One  -.-  -.

Darrell, VE1ALQ

-----------------

If you have any other sorce information, PSE let me know. I'll include it

for those who has difficulty in their local parts shop.

de Sueo,  JA6FTL