PRACTICAL CONSIDERATIONS

FOR 9600 BAUD OPERATION

Compiled by WÝTN

(mostly from TAPR netsig)

Rev. 2, 12-30-94

Talking with Roy N5RG today he said something that is right.  We

were talking about our 9600 baud modems and radios. And about only 2

other 9600 baud stations. Roy said that while information theory and

experiance tells us to run 1200 baud on the long runs but make the local

links much faster, like 9600 baud.

The problem is that the ordinary ham isn't capable of making 9600

baud work. The 2 stations in El Paso, TX which are run not as nodes but

as personal JNOS connects to the network are having lots of problems.

This is not do to weak signals. They both have line-of-sight to the node

#ELP. But they are not Engineers and don't have an oscilliscope to

display eye waveforms or a Service Monitor to set frequency and transmit

deviation. So they are not able for reasons of training and equipment to

get 9600 baud systems working.

So lets be reasonable and admit that the new No Code Ham is not

going to have a scope, or freq. meter or any of that stuff. But if we are

successful we infect them with a desire to try TCP/IP and packet radio in

general. These fine people will buy a tnc and with lots of trouble get it

connected to the new HT they just got.

At 1200 baud it's a good guess the radio will be fine for the

narrow 1200 baud spectrum. They get it running and begin to learn about

packet and computers and the fun. After awhile they may want to try a

faster system. We must either get smart and design a 2400 or 4800 baud

modem that will ALSO run fine with the HT, and give them more speed. But

we must be honest with them and say NO! Do NOT buy a 9600 baud modem!

-karl k5di

---------------------------------------------------------------------

[I'm re-directing this thread to NETSIG@TAPR.ORG . Please do not follow-up]

[to TCP-GROUP@UCSD.EDU, there is no reason to use _two_ lists. - Bruce]

Karl Larsen said:

> Roy said that while information theory and experiance tells us to run

> 1200 baud on the long runs but make the local links much faster,

> like 9600 baud.

You must mean user-to-BBS connections, as 1200 wouldn't be practical for

a backbone.

> The problem is that the ordinary ham isn't capable of making 9600

> baud work.

I made 9600 work with only a voltmeter. I used off-the-shelf radios

and TNCs (TEKK and Kantronics), which makes a big difference. I used a

$17 Radio Shack TV antenna and a 4:1 coaxial balun. I wouldn't expect a

new ham to convert a Micor. I set the voltage to the input of my TEKK to

the one I was told would make it deviate about 3 KHz. It worked. Big deal.

> But they are not Engineers and don't have an oscilliscope to

> display eye waveforms or a Service Monitor to set frequency

> and transmit deviation. So they are not able for reasons of

> training and equipment to get 9600 baud systems working.

Kantronics says to measure the voltage at the BER test point of their

TNC, as it is more accurate than eye patterns. If I had a TAPR modem

I'd have no choice but to look at eye patterns.

> So lets be reasonable and admit that the new No Code Ham is not

> going to have a scope, or freq. meter or any of that stuff.

OK. They don't need them.

Also, you are completely discounting the fact that the world is full

of Elmers. I don't have a service monitor (though I admit to lusting

for one) but when I really need one, I can drive over to a buddy and

use his.

> We must be honest with them and say NO! Do NOT buy a 9600 baud modem!

I wouldn't suggest any new ham buy the TAPR modem, but there is good

off-the-shelf equipment now.

The problem with running a LAN at 9600 with these two-watt radios

is that you need a bit-regenerating repeater to eliminate the

hidden-transmitter problems so that everyone can use directional

antennas pointing at the repeater, everybody can hear each other,

etc. So let's build repeaters to make it easier for the naive user

to join a LAN.

Karl, a while back you were arguing why it was necessary for us to

build software for people who could not afford a 486. Now you have a

486 and I don't hear that argument any longer. Could it be that you're

going to drag your feet on 9600 for another two years and then join

that bandwagon too?

Thanks

Bruce AB6YM

------------------------------------------------------------------------

My mailer said LARSEN KARL AMSRL-SL-EC 505-678-3145 said this:

>

> 	The problem is that the ordinary ham isn't capable of making 9600

> baud work. The 2 stations in El Paso, TX which are run not as nodes but

[stuff deleted]

> 	So lets be reasonable and admit that the new No Code Ham is not

[more stuff deleted]

Well lets see,  I have 3 MFJ tnc's here all with 9600 baud modems. I

bought one tnc and immediately opened it up and installed a Paccomm modem

without ever hooking up the tnc.  All 3 work fine with no O-scope or any

equipemnt whatsoever to test it out.  After they were hooked up to radios

(I have 3 TEKK rigs here for testing and also a  Kenwood TM-731A dual-band

mobile modified on both VHF and UHF for 9600)  and all have worked great

the first time. Afterwards I hooked up a scope and found them to be right

on.   I do require about an S3 signal though to maintain a totally reliable

9600 link. My Kenwood will decode 9600 almost error free with an S1 signal

on it's scale (who knows what it really is though).  Setting up a 9600

tnc is pretty much idiot proof and anybody that can pass a ham test SHOULD

be knowledgeable to do this.  If they just memorized the questions and

answers in the back of the study guide and never learned any of the theory,

then my attitude is that it's their problem.  I learned the theory about

16 years ago and know people that learn it know. Too many people just

memorize Q&A and then start whining that they don't understand something

as simple as installing a 9600 modem in a tnc.  I can put on in and have

it on the air within a half hour to an hour.   With new rigs coming out

that are supposed to be 9600 baud ready,  and tnc's out with 9600 modems

in them,   if they can't get that going then it's more likely 'operator

headspace'.

There are lots of 9600 baud packet folks around here that are not at all

technically inclined. They just bought a DRSI 9600 setup (DRSI TNC and

Alinco radio) and they took it out of the box, plugged it in, and it worked

immediately and within spec and proper levels right out of the box.

Anybody and everybody can do 9600. They should be more knowledgeable than

to try to use a lousy HT on packet anyways. If they ask why then tell them

to read the theory that they are required to know to get their license.

--

Ron N8FOW

AMPRnet  : n8fow@n8fow.ampr.org

Internet : ron@chaos.eng.wayne.edu

aa011@detroit.freenet.o

--------------------------------------------------------------------------------------

Based upon the comments I got from the guys on this subject my

Doberman buddy Daisy should be able to buy $500 worth of gear and get on

9600 baud no sweat.

All you need is a screw driver. And all you need that for is to

open the box...

There may be today Kantronics or other hardeware you can buy that

just works. It has been tuned up at the plant and sent out. But it's my

experiance with Paccom and tapr modems that the hard part is:

1. Getting the modem-to-radio cables made and levels set properly.

2. Getting the radio on the right frequency(s). The hill top

nodes may NOT be on the published frequency. Many 1200 baud nodes are 500

to 1000 Hz off frequency and work fine. But that far off at 9600 baud

will seriously degrade performance.

So if there is someone that thinks we all can just put 9600 baud

modems into tnc's and wire them up and go are deluded. I have worked with

4 nodes using both Tekk and Mitrex radios. The tnc's are tiny-2 from

paccomm with the g3ruh modem from paccomm. They are all working just fine

now...but it takes a lot of tweeking to get them working right. I have

one connected to my g8bpq switch. That tells me the re-try rate. I was

running 250% retries before I tuned my receiver frequency 200 Hz closer

to the frequency being used by the other node. Now it is running less

than 50%.

I plan to get it down to 0% but that takes a lot of work. I think

today I will skip it. What do you think a non-technical ham will do?

-karl

---------------------------------------------------------------------------------------

On Sat, 29 Oct 1994, Klarsen wrote:

> 	So if there is someone that thinks we all can just put 9600 baud

> modems into tnc's and wire them up and go are deluded. I have worked with

> 4 nodes using both Tekk and Mitrex radios. The tnc's are tiny-2 from

> paccomm with the g3ruh modem from paccomm. They are all working just fine

> now...but it takes a lot of tweeking to get them working right. I have

> one connected to my g8bpq switch. That tells me the re-try rate. I was

> running 250% retries before I tuned my receiver frequency 200 Hz closer

> to the frequency being used by the other node. Now it is running less

> than 50%.

Yeah, but then you tell the rest of us on the packet channel our nodes

aren't working. Instead of chasing the mountain top TEKKS around the

band, I say the hams should stay on the published frequency. Heck if we

chased a TEKK up and down the bands we'd be all over the place. I

wouldn't be surprised if the reason our RF problem is soo bad is because

you and the mountain top node are off frequency. I don't have time to

chase everyone all different directions around the band. I had enough of

that when I operated my Hallicrafters (ole drifty!) on 20 meters RTTY :-)

> 	I plan to get it down to 0% but that takes a lot of work. I think

> today I will skip it. What do you think a non-technical ham will do?

Get it on 445.1 MHz, 3.4 KHz deviation, with a good eye pattern going

into the transmitter and leave it alone.

73

Tim, AA5DF

Tim@NMSU.EDU

------------------------------------------------------------------------------------

Karl,

I'm not a shoe salesman. Did you see the restored version of "Snow

White"? I wrote some of the software used to clean up every frame of

that film. However, I don't know _all_ that much about RF, so I figure

I'm representative of little-experienced operators in that area. I'm

nearly innumerate by the standard of people who work at Pixar - my

software doesn't do much more than multiply and add. I do have a good

grasp of systems programming and networking protocols. I don't own the

equipment I'd need to work on RF, even if I understood it.

So, given that background, what I did was take two radios and two

TNCs in the same room, set the TNCs up to provide about the right

voltage (not using a RMS voltmeter, either) to the radio inputs,

and tweaked the input level pots a bit while watching the voltage

on the BER outputs of the TNCs. No big deal.

After that, I went to Radio Shack and bought one of their UHF TV

receiving antennas because it "looked right" for 433 MHz from what

I'd read about antennas. I cut a 4:1 coaxial balun out of coaxial

cable and used it to match the antenna. It worked. No big deal.

However, can I use that like a 1200 Baud 2 Meter TNC in a LAN? Not

with two watts. I could if there were a bit-regenerating repeater.

So I'd suggest higher power if you don't have a repeater at the center

of your LAN, and even that won't work as well as it would with the

repeater in place.

The appliance operators will probably wait until we've done the

pioneering to them. I would suggest to beginners who aren't appliance

operators that they get 9600 equipment, so that they can have the pride

and experience of being pioneers and working out the problems for those

who will come after them.

Bruce

------------------------------------------------------------------------------------

> However, I don't know _all_ that much about RF, so I figure

>I'm representative of little-experienced operators in that area.

I find it interesting that you can count the 9600 baud users

in all of Northern California on less than both hands.  What's

that say about the level of experience?

--

*--------------------*----------------------------------------*

|  John Zantek WX3K  | Internet: jzantek@tcpet.uscg.mil       |

|    [44.4.40.104]   |   Packet: WX3K@WX3K.#NOCAL.CA.USA.NOAM |

|  Data 707 765 7016 |  Amprnet: wx3k@w2air.ampr.org          |

*--------------------*----------------------------------------*

------------------------------------------------------------------------------------

William Barnes asked me to say something about the Radio Shack TV antenna

on 440. The antenna is their "75 mile" UHF-only antenna, and it cost $17.

It's a corner-reflector with directors. I don't know if the directors

are working but the corner reflector works. I cut a 4:1 coaxial balun

from instructions in the ARRL Antenna Book. That's all I'm doing right

now and it seems to work well. I didn't trim elements and I have somewhat

of a mis-match. Someone with the right tools could trim the elements,

perhaps lengthen the directors, and get a few more dB out of it. For $17

it's cheap fun for an afternoon.

Bruce

------------------------------------------------------------------------------------

On Sat, 29 Oct 1994, Klarsen wrote:

> 	So if there is someone that thinks we all can just put 9600 baud

> modems into tnc's and wire them up and go are deluded. I have worked with

Yup I do. You want proof, come to Detroit.  We even have people here that

wanted to get into packet and their very first packet system was a 9600 baud

one. They have never owned 1200 baud and have no reason to.  They are also

not into the technical scene too.  They open the box, plug it in, and it

works the first time.  You must have something wrong if you can't get it to

work.  I know that some radios need modifications, but newer ones don't.

A TEKK micro should work out of the box, but I had to make a few mods on

the 3 Minis here to get them to work.  For new users,  don't buy a radio that

requires mods,  that's what most folks around here do.  I'm one of the few

people in the area that 'built my own'  9600 setup.

9600 is user ready today and it has been.  There isn't much need for an

arguement anymore about it since we have the proof right here in Detroit

and many other cities with first-time packet folks using it.  Now 19200

may be a different story, how about that speed?

--

Ron N8FOW

AMPRnet  : n8fow@n8fow.ampr.org

Internet : ron@chaos.eng.wayne.edu

aa011@detroit.freenet.org

-----------------------------------------------------------------------------------

>On Sat, 29 Oct 1994, Klarsen wrote:

>> 	So if there is someone that thinks we all can just put 9600 baud

>> modems into tnc's and wire them up and go are deluded. I have worked with

>Yup I do. You want proof, come to Detroit.  We even have people here that

>wanted to get into packet and their very first packet system was a 9600 baud

>one.

FYI..

If you purchased a chrome case TEKK (mini/micro I forget) in 1993

or the first few months of 1994 you are guaranteed to have problems

at 9600. Later units should be modified to bypass the limiter and

improve the frequency response. Maybe this is responsible for poor

results vs. "plug & play" results! (I believe this is also true for

early TEKKs shipped my PacComm) For the mods, check the posting I

made last week.

Getting on 9600 with a TEKK and a TNC is a lot easier than I once thought.

I don't believe it's a no-brainer (I have heard of hams that couldn't

get 1200 baud TNC's hooked up. The average ham these days should

be able to purchase a TEKK (already modified) and a TNC, make a cable,

and use it. To make tx adjustments, I plan to install a deviation meter

in our 9600 bps user node. That addition in our 1200 baud node has

helped countless hams.

> Now 19200 may be a different story, how about that speed?

>

>Ron N8FOW

Has anyone modified TEKKs for 19.2 Kbps? It looks like the

TX is ok. I'm looking for drop-in 1st IF crystals right now.

John, WB3ESS

-----------------------------------------------------------------------

>Where can I get a compiled list of radios that work on 9600 and the

>required MODS. I am putting together a "how to" for 9600 and need to

>include it. Preferably in digital form, but if not I could scan it.

>

>Doug

Mike Curtis, WD6EHR, has put together a document containing much of this

type information.  I believe he is not currently on the Internet, but a copy

of the latest version is on tcet.unt.edu as /tapr/9600/96man2x0.txt.  This

is obviously not all-inclusive, as the number of 9600-capable radios has

grown much recently, but it does cover mods to quite a few of the

commonly-used radios.

Bob

-------

Bob Nielsen, W6SWE

Tucson, AZ

nielsen@primenet.com

-----------------------------------------------------------------------------------

To: NETSIG1 --INTERNET TAPR NETSIG

FROM: David C. Bray             dcbr@chevron.com

CITC/CSD        CTN 675-3838 Area Code 303

----------------------------------------------

Subject: MFJ-9600 modem modification needed.

Hi All,

I have been working with other node-ops around my area to

assist them with setting up 9600 baud links, and have found a

disturbing characteristic of ALL FIVE of the MFJ-9600 modems I

have come in contact with.

If the transmit output level on the modem is set to anything

over the lower 1/3 or 1/2 of the adjustment range the signal

becomes very distorted. I have determined that the source of this

trouble is when the signal passes through Q4 and Q5 on the modem

board. I have been bypassing these transistors by removing C34

and C37, connecting them in series, and re-inserting them back

into the board so that the signal will still go from U10 (pin 1)

to the output of the modem. The only reason I can see that Q4 and

Q5 are there in the first place is to remove the TXD line from

PLL radios when the oscillator you are modulating is also a

reference for the receiver.

When this change is made, the modem gives a clean output

signal, and is working well in our network at all five locations

where these modems are in place.

I have successfully interfaced this modem with Motorola Micor

UHF mobile, Motorola Maxar UHF mobile, and Motorola Syntor UHF

mobile radios.

I hope this helps all of you out there.

73s   -Dave-  N0ITS @ KB0CZV.#NWCO.CO

email:  dcbr@chevron.com

-----------------------------------------------------------------------

Date: 1 Nov 94 21:13:00 GMT

From: steve.diggs@totrbbs.atl.ga.us (Steve Diggs)

Subject: 9600bd radio's ?? NOT !

-> From: joopv@etprs.phys.tue.nl ()

-> Subject: 9600bd radio's ?? NOT !

-> I am wondering if anybody did the same tests on so-called '9600bd

-> ready' transceivers. Especially the FM rigs, the allmode rigs should

-> be ok because they have a separate FM modulator, and don't use the

-> PLL for modulation.

Hi Joop,

The East Atlanta LAN, a packet experimenters group here in Atlanta, has

begun operations with our first general purpose 9600 digi. While we

haven't done a good statistically based RF performance test of various

transcievers yet, our experience with the "9600 ready" rigs has been

good - when used with a TNC that has a sound G3RUH implementation to

start out with. The two factors are inexorably tied; bad modem/radio

matchup; poor performance. The Paccom Sprint2 is an excellent performer

in terms of low distortion in the transmitted eye to the xcvr; so is the

PK232/TAPR 9600, KPC9612, and KPC Data engine. The MFJ1270 + their G3RUH

implementation is just terrible. I maintain that one of the sound TNC

performers noted here with a "big three" 9600 ready xcvr. will produce

better-than-acceptable performance on-air.

-> And what about those turnaround times ? 120 ms is IMHO totally

-> unusable for 9600bd packet. This thing sure is going to produce lots

-> of collisions on busy channels !

Boy, are we really operating in the blind! We operate with TXD set to 22

and it performs beautifully! Based on what you say here, I'm going to

cut it down to 13-15 and see how it does!

-> I am wondering if anybody did the same tests on so-called '9600bd

-> ready' transceivers. Especially the FM rigs, the allmode rigs should

-> be ok because they have a separate FM modulator, and don't use the

-> PLL for modulation.

Our experienc has born this out. I have a Kenwood TS811 which has been

modified for 9600, and we kind of use it as the standard to measure

other radios against. I have been trying to get the cheaper PLL

modulated rigs to work for a year and a half - with disappointing

results. About 3 months ago, James Miller suggested a new approach to

me: try a "two point modulation" approach, wherein both the VCO AND the

master reference oscillator are modulated with the 9600 audio. In this

manner, the cancellation effect of the PLL is eliminated. I have even

found a company, MX*COM, which makes a chipset to facilitate this

approach. I haven't had time to try it, but it's on this winter's list

of projects. If such an approach were to work, it would open 9600 packet

to MANY, MANY hams who own run-of-the-mill FM only rigs. Any ideas here?

I look forward to your feedback, hoping that we can open this field up a

lot of creative work!

Regards,

Steve Diggs, President

East Atlanta LAN

----

Top Of The Rock BBS - Lilburn, GA         SYSOP: Steve Diggs

UUCP: totrbbs.atl.ga.us               Snailmail: 4181 Wash Lee Ct.

Phone: +1 404 921 8687                           Lilburn, GA 30247-7407

------------------------------------------------------------------------

> From: dreamer@lhaven.uumh.ab.ca (Lawrence *The Dreamer* Chen)

> Can the KS-900 be used for 1200 baud and 9600 baud at the same time

> easily?

Your 1200-baud TNC _must_ be capable of running open-squelch to use the

TEKK radio. The KPC-9612 has that capability and should always be run

open-squelch. One of the TEKK radios has a DCD output, but it is not

fast enough to be useful.

[What does DCD have to do with the radio?  DCD is a TNC function.  Perhaps Bruce means the TEKK receiver has no squelch? -W0TN]

I modified an Alinco DR-1200 radio for 9600 operation, and have since

been using it for 1200 with a _modified_ AEA PK-88. The PK-88 had to be

modified with the TAPR state-machine DCD board, because the 9600

modification bypasses the squelch. The PK-88 had a software command to

turn off the equalization it would have used were it running on the

microphone and speaker outputs. That equalization is usually necessary

because FM voice radios use pre-emphasis and de-emphasis to change the

noise spectrum of received audio to something that is less offensive to

the ear and improve the intelligibility of the received audio. The 9600

modifications bypass pre-emphasis and de-emphasis.

The KPC-9612 is capable of running 1200 and 9600 on the same radio. I

suspect that 9600 packet will not reliably activate the DCD

(carrier-detect) of every TNC on your 1200-Baud network, and

vice-versa, and thus there is less chance for peaceful co-existence

than you might think. Moving to a new frequency for 9600 seems to be

the accepted strategy.

Thanks

Bruce Perens

----------------------------------------------------------------------

On Fri, 11 Nov 1994, Klarsen wrote:

> 	Bruce the touch-tone method of setting deviation is not a joke. I

> first took my ht to a service monitor and had it's touch-tone deviation

> measured (it is almost exactly 3 KHz).  I then calibrate my scope image

> in terms of p-p deflection with the ht making 3 KHz sinewaves. Then I

> look at the signals I am connected to and note their deflection. Using

> this method I discovered that the mountain-top node has a 1 KHz deviation

> now and I will get up there on Monday next to fix it.

>

> 	I should say the scope is connected to the Tekk receiver audio output

> line. Any DC scope will work. This is as accurate as it gets Bruce. The

> only thing better is a $12,000.00 service monitor...

>

> -karl

>

Karl,

I have used a modified version of your method for years and it is

very accurate. Instead of using the touch tones to calibrate the scope,

I use a ht that can transmit 5khz above the frequency you are using and

5khz below. Using the variable gain control on the scope set the +-5

signals to convient graticle marks. My scope sets up to +-3 divisions to

indicate the +-5KHz offsets. Then a packet signal filling 4 divisions on

the scope is at 3.333khz deviation.

I once had an FCC engineer tell me that this was exactly the method

they use to calibrate their deviation monitoring equipment. Of course

this assumes that the monitored discriminator is somewhat linear and you

are dc coupled.

I keep a modified Bearcat scanner on the workbench just for deviation

monitoring. Just tap onto pin 9 of the receiver IF chip, just like you

would do when modifing a rig for 9600 baud use, and run it out to a

chassis mounted bnc connector. Works great! It is very convincing to

show a new packet users that their poor results are because of over

deviation and the resulting square waves being transmitted on 1200 baud.

Sperry/n4sd

n4sd.ampr.org [44.62.64.40]

sperry@pinn.net

------------------------------------------------------------------------

Sperry N4SD commented about his modified bearcat scanner being used with a

scope for deviation measurments.  I too have used this technique for many

years with good results.  AC coupled like Karl K5DI suggested with the

TEKK will do a nice job on deviation.  DC coupled as Sperry suggests will

give BOTH frequency and deviation indications.  Just by glancing at the

scope, one can tell right away whether a CARRIER is on or off frequency and

by how much.

Just about everyone now has a scanner in the shack and this is a convenient

way for a network manager to keep tabs on those nodes he is capable of

receiving.  Other than figuring out how to DC couple to your particular

scanner, the major point is to have a reasonably trusted calibration source

at hand with which to check and set the scope amplitude.

I use an old HP-130 scope for this purpose.  It has a 5 inch screen is

quite obsolute by todays standards, and as a result can be found cheaply.

^ obsolete

73 de Jack

---------------------------------------------------------------------

Date: Sat, 12 Nov 1994 13:20:04 GMT

From: usenet@mv.mv.com (System Administrator)

Subject: 2M 9600-ready radios; which would..

In article <3a1jeb$hsl$1@mhade.production.compuserve.com>,

74212.1733@CompuServe.COM says...

>

>The best Radio for 9600 use today is the Motorola Mitrek!  The

>JAP crap is just that!  Too slow!  73!

>Dino...KC6RIX

I understand the Motorola Moxies and Maxars are equally good and fairly

inexpensive.

Larry W1HJF

----------------------------------------------------------------------

Date: Thu, 17 Nov 1994 13:34:37 GMT

From: rapp@lmr.mv.com (Larry Rappaport)

References<nickbCz0L0M.Hrv@netcom.com> <3a1jeb$hsl$1@mhade.production.compuserve.com>, <3ae5j6$d5r@nntp.interaccess.com>

Subject: Re: 2M 9600-ready radios; which would..

In article <3ae5j6$d5r@nntp.interaccess.com>, pgreen@interaccess.com

says...

>

>Dino Darling (74212.1733@CompuServe.COM) wrote:

>: The best Radio for 9600 use today is the Motorola Mitrek!  The

>: JAP crap is just that!  Too slow!  73!

>: Dino...KC6RIX

>

>Yep,  I have a Mitrek.  A buddy of mine has one too.  I can reliably pass

>data to him with TXDs as low as 60 ms.  With a TXD of 150 the thruput is

>99%.  This is on 2m.

>

>Pat --- KA9SCF.

>

>pgreen@interaccess.com

Isn't 60ms a bit high?  I was under the impression that the Motorola

Moxie (Moxy?) could do it at around 25ms.  Supposed to be a good,

inexpensive radio for that purpose.  My information is all second hand,

however.

Larry W1HJF

----------------------------------------------------------------------

In article <CyuKvo.15p@metronet.com>

kf5mg@metronet.com writes:

> In <398hk6$8pl@bingnet1.cc.binghamton.edu>, bd27015@bingsuns.cc.binghamton.edu (Phlatline) writes:

> >This is kind of a plug on from an earlier author's message.

> >

> >I saw an ad for MFJ's dataradio which is supposedly 9600 baud ready.

> >It's a standalone crystal controlled system. (one frequency at a time,

> >probably allows simplex/duplex with appropriate crystals.)  any way

> >the long and short of this is that i am lookin to get into packet (and

> >120 for this radio does not seem like a bad price) and what ever speed

> >i choose to start at i would like to be able to go to 9600 at any time

> >i want.....

> >

> >any comments on this radio please mail or post here

> >

> >--Dave Graff

>

> Call up MFJ and tell them you want info on the radio. I called them last

> week to get some info on the radio. They're not ready to ship yet. The

> radio requires some crystals with very strict tolerances and they're

> waiting to see what freq. people will want to run 9600b on. I suggested

> 145.53, 145.67 and 145.69. I asked them to send me info on what type of

> crystal was needed but I haven't received that info yet.

>

> 73's  de  Jack  -  kf5mg

> Internet        -  kf5mg@kf5mg.ampr.org            -  44.28.0.14

>                 -  kf5mg@metronet.com              -  work (looking  for)

> AX25net         -  kf5mg@kf5mg.#dfw.tx.usa.noam    -  home (817) 488-4386

> +=======================================================================+

> +                   D.A.M. - Mothers Against Dyslexia                   +

> +=======================================================================+

>

------------------------------------------------------------------------------------

The process of evaluating data modems and radios appears to verge on the

black arts.  All too often the main recourse the average packeteer has

is too hook up his gear and see if it "works".  I feel there is a BIG NEED

to have someone(s) write an article(s) for the PSR explaining in simple terms

what test equipment and procedures are needed to perform one's own

evaluation on his equipment.

Most of the measurement procedures are "out there", but finding and setting

up to use them can be rather frustrating if one hasn't gone thru a

particular procedure before.  Awhile back I was curious as to what phase

delay changes were taking place before and after some IF filter mods.

After some search, discovered my old Tektronics scope manual explained

this application in sufficient detail for me to grasp and to perform it

with reasonable precision.  Now my reference library may not be up to

date, but it is reasonably extensive...and except for the tek manual,

silent on making phase measurements.

Another problem in this area is sematics.  Test parameters should be

clearly and consistently defined.  I wrestled quite a bit with the terms

associated with "delay".  My references conflicted in the definition of

"group delay".  It finally dawned on me that in the context used within

our packet circle it was meant to indicate the amount of change in phase

over a limited range of audio frequencies.

If we are EVER to see packet move "forward" those who understand how to

make the typically few test configurations, needs to take the rest of us

by the hand and walk us through it.  Once, more folks within our ranks

feel confident enough to evaulate the performance of their gear, SOME of

these people may start coming up with worthwhile mods and design changes.

As an example of this theme I've extracted the following from rec.radio.

amateur.digital.misc:

Subject: 9600bd radio's ?? NOT !

Date: 1 Nov 94 20:46:19 GMT

Organization: Eindhoven University of Technology, The Netherlands

Lines: 63

NNTP-Posting-Host: etprs.phys.tue.nl

Summary: Are '9600bd ready' radio really ready ?

Keywords: packet, 9600bd, g3ruh, k9ng, amateur-radio

X-Newsreader: NN version 6.5.0 #11 (NOV)

Since some time now several manufacterers of ham radio equipment offer

'9600 bd ready' vhf/uhf transceivers. I got one of these for some tests,

and was very surprised with the results. This is a brandnew mobile uhf rig,

FM only, from one of the 'big three' brands. The price is in the $500-$600

range.

Turnaround time : TX to RX is 120 ms.  RX to TX is 140 ms

Transmitter frequency charasteristics : (reference = 3kHz deviation at 1kHz)

160 Hz -  7400 Hz straight whitin 2 dB. (160 Hz : +1 dB, 7400 Hz : -1 dB)

When the freq. drops we see this peak :

95 Hz : +3   dB (4.2 kHz deviation)

59 Hz : +6   dB (6   khz deviation)

41 Hz : +8.5 dB (7.7 kHz deviation)

30 Hz : +6   dB (6   kHz deviation)

25 Hz : +3   dB

22 Hz :  0   dB

20 Hz : -3   dB

I also measured group delay time from the transmitter :

30Hz : -11 ms, 35Hz : -7.5 ms, 41Hz : -4 ms,   45Hz : -3 ms,   45Hz : -3 ms,

50Hz : -2 ms,  60Hz : -1 ms,   70Hz : -480 us, 80Hz : -230 us, 90Hz : -100 us,

110  Hz : 0 us, 150  Hz : 50 us, 500  Hz : 50 us, .... 7000Hz : 47 us.

I also did the same measurements on the receiver. There the frequency response

was totally flat from 12 Hz to 5 kHz. -6db points at 10 Hz and 7 kHz.

Group delay was at a constant 150 us over the complete 20 Hz to 7 kHz range.

The next thing was of course to do some packet tests with 9600bd modems.

I did it by transmitting and receiving >10000 full-length packets

of 256 bytes.

The result was astounding, but understandable when you look at the above

measurements : receiving was ok, about 98 % of all packets were decoded ok.

But the transmitter produced only 55 % readable packets !!

This is clearly a PLL-transceiver which is modulated at the VCO from the PLL.

How can it be that this rig is being sold as '9600bd ready' ? Do they have

different 9600bd modems in Japan? The G3RUH / K9NG modem system needs a flat

frequency response from 20 Hz to 6 kHz.

And what about those turnaround times ? 120 ms is IMHO totally unusable

for 9600bd packet. This thing sure is going to produce lots of collisions

on busy channels !

Or am i setting my standards too high ? For somebody who only receives large

packets and transmits small packets ( the average mailbox user ?) the rig

may *seem* to work ok. (at a given bit error rate, the chance of producing

a garbled packet is of course smaller for small packets)

I am wondering if anybody did the same tests on so-called '9600bd ready'

transceivers. Especially the FM rigs, the allmode rigs should be ok because

they have a separate FM modulator, and don't use the PLL for modulation.

In fact, i tested an allmode rig from the same manufacterer and found it to

be excellent, turnaround times of less then 25 ms, and flat frequency

responses. And thus also very low bit-error rates, 97 % or better according

to the above-mentioned tests.

Joop, PE1DNA.

End of forwarded message -

73 de Jack

----------------------------------------------------------------------

Date: 23 Dec 1994 09:51:11 GMT

From: ron@chaos.eng.wayne.edu (Ron Atkinson)

References<3d562m$4qc@gdls.com> <3d8fd1$o5l@oak.oakland.edu>, <1994Dec22.215951.29654@seas.smu.edu>

Subject: Re: line modems for amateur use?

Kenneth J. Hendrickson (kjh@seas.smu.edu) wrote:

: The latency of a delay before sending data over a transmitter, to wait

: for stabilization etc., will be made negligible if the packet lengths

: are increased to a large enough value.  As bit rate increases, packet

: length ought to also increase so as not to be swamped by the startup

: latencies.

First off,  ignore my last post of resending your message, goofed up

my follow-up. I'm entitled to a mistake a year, I'll use mine now :-)

After beating on 9600 for a while I found a few things about packet

sizes and txdelay.  First the biggest factor in running from the

mid 400 cps range to going up into the 600-650 cps range was a simple

switch from 9600 to 19200 on the serial port.   Another way to increase

it is to run larger packets. I'm running a tcp mss 1024 and tcp

window 2048  now and thruput is faster on ftp's.

The above tests though are with your 'not-so-normal' everyday packet

use. That was by doing a lot of ftp's of various sizes of files. In

normal use though the single biggest factor in response time over packet

was always the txdelay.  Someone running 300 ms seems pretty sluggish

at 9600, but 80 or 100 ms seems a lot faster. It's the smaller packets

that are sent in normal everyday use such a wandering around a BBS,

sitting on a DX cluster, keyboard-to-keyboard.  When you go up into

higher speeds your txdelay may become longer than the transmission

of actual data, and this is what makes it start appearing sluggish.

On a relatively continuous transmission over a period of time, like

an ftp, varying the txdelay from 100-300 ms doesn't seem to make a

whole lot of difference though,  it's the packet size that does it

there.

I got pretty strict on txdelay around here.  I had a lot of people say

"my radio can't use a fast txdelay".  These are folks running modern

radios too.  I finally sat down on a busy freq with some friends and

I found that when I ran 150 ms at 1200 baud even an Icom O2 HT running

closed squelch will decode every packet with no problems.  Most radios

I ran across worked fine at 150 ms, even HT's did.  Of course our

tnc's were all calibrated correctly too and we weren't runing high

audio levels or deviation eithers.  The brand of radios though that

did have the biggest problems were always the Alinco radios. If someone

starts to miss packets from me my first question is always "Are you

running an Alinco?", and they almost always are.  Open squelch doesn't

help either, it's the radio.  My Kenwood TM-731A dual-band is the

fastest mobile rig I found. I can transmit with 80 ms and receive

at 50 ms txdelay.   I now run 150 ms at home and 200 ms on a switch

site for 1200 and 100 ms for 9600 and I wont' go higher either, but I

will go lower with them.

Ron N8FOW

------------------------------------------------------------------------