Comments On

COORDINATION OF DIGITAL REPEATERS

Collected By WÝTN

Rev. 2, 1-4-95

The following was posed to the TAPR NETSIG and Internet rec.radio.amateur.digital.mics newsgroups:

I'm on a newly appointed Minnesota Repeater Council (MRC)

committee charged with preparing a report and recommendation

relating to coordination of digital repeaters on "traditional" FM voice

repeater channels (pairs).  I believe the MRC will coordinate digital

repeaters on those frequencies if it can be shown that the digital

repeater operation will not cause interference to adjacent and co-

channel voice repeaters.  (The current coordination policy specifies

geographic separation distances for adjacent and co-channel voice

repeaters based primarily on frequency band.)

To help us prepare a coordination policy for digital repeaters I would

like to hear from others with experience in this area.  In particular, I

would like to identify specific cases of digital repeaters co-existing

with voice repeaters, the technical and coordination conditions

affecting their interactions, and the results.

Your inputs will be greatly appreciated and can help us increase

digital resources for operators in this region.

Replys are collected below.

-------------------------------------------------------------------

Boy, my first impression at reading this was "Are repeater coordinating

councils so ignorant?  Of course there is no interference.  Packet lives

within the same bandwidths as voice."  Then as I thought about it some more,

perhaps this might be part of the stumbling block of coordinating voice

frequency pairs for data.  Rick, you are off to a good start and the

information you gather can hopefully be used in other areas with the same

problems.

I know that the repeater coordinating group for Northern California won't

hear of having data on a "voice" repeater frequency pair.  Thus keeping data

in it's part of the frequency band plan.  What ever happen to the days of 2

meter RTTY repeaters?  Lest we forget.

73,

Steve, N7HPR

srbible@cs.nps.navy.mil

Naval Postgraduate School,  Monterey, CA 9394

-----------------------------------------------------------------

Hi Rick - you should contact the Texas VHF/FM Society.  We have a bandplan

in place for digial repeaters.

In addition, digital repeaters in the voice repeater are handled the same

way -- except that they make sure they don't fudge on the repeater spacing.

The main reason for this is that since packet operates nearly

continously....a band opening can really cause problems.  The VHF/FM Society

treis to get voice repeaters to operate with Toned Open-Sql.  In this way

the repeater can leave there sql fairly tight and those on the firnges use

tones to get the sql to open fully for them to make it.

Greg Jones

-------------------------------------------------------------

The Pikes Peak FM Association operates a 1200-baud 2m repeater on 145.205,

which is a 'controlled access pair' within the Colorado voice repeater

bandplan.  It is coordinated to the extent that when the machine was first

built, the coordinating body thought it a bad idea to put it in the middle of

the voice repeaters that were active, but thought way out on the end of the

controlled access pairs would be "ok".  Sigh.

Since this machine was perceived as successful, we've since built a 9600 baud

70cm repeater on 447.250, which is smack in the middle of our voice repeater

allocations.  It is a fully coordinated machine.  The Colorado Repeater

Association has just deployed a 9600 baud 70cm repeater elsewhere in the

state on 447.200, co-located with their voice repeater on 447.150 sharing the

same antenna system.  Our machine has been perceived as a great success, the

CRA machine is too new to judge.

The real key is to try and ensure that *all* repeaters being put on the air

on coordinated frequencies, whether voice or data, are well-built and well

maintained.  It isn't always obvious how to orchestrate this, but a poorly-

designed or poorly-maintained RF deck can splatter and annoy the neighbors

regardless of whether the mode is voice or data.

The fact that our digital repeaters were built and deployed by established,

respected voice repeater groups probably helped in the coordination process.

They're all Motorola RF decks, tuned and bench-qualified with the same

processes we use for our voice machines...

73 - Bdale, N3EUA

Chairman, Pikes Peak FM Association (among other things

-------------------------------------------------------------

Rick:

The idea of sharing channels between voice and voice-bandwidth digital

systems sounds like a good idea.  My only question is exactly what do you

mean by a "digital repeater."  A bit regenerator or something similar?

That can closely match the voice repeater paradigm, but (as I have said

before, from atop my soapbox) routers (a-la-KA9Q) make a lot more sense.

They don't repeat the signals that don't need to be repeated, and

don't require a freq pair (so 2 routers can use a single "repeater pair").

Throughput may be a bit less when it does re-transmit packets, but it

will re-transmit fewer packets.

That assumes that everything is IP-based, as it should be :^)

David, N0OXH, rush@erg.sri.com

---------------------------------------------------------------

One of the loud voices opposing digital on "voice" frequencies is a well-

known local Amateur who complains bitterly about not being able to

"band-scan" because of the packet bursts!

[Some info deleted]

Carl (WA0CQG)

----------------------------------------------------------------

Hi Rick,

Here in the Seattle area our local repeater coordinators (Western Washington

Amateur Relay Association) has coordinated several repeaters for digital use

in the traditional 'voice' band.  There is one 9600-bps machine on two

meters, a 1200-baud system on 222 MHz, and 3 on 440 MHz.

To my knowledge the only problems we've had with adjacent channel

interferece have been on two meters.  The repeater in question is on

146.38/146.98 MHz and has a 146.36/146.96 and a 146.40/147.00 repeater

within 30 (or so) miles of it.  I happen to be involved in both the packet

repeater, as an owner, and in the 146.96 repeater, as a member of the owning

club, and builder of the repeater.  The packet repeater is on a 5000'

mountaintop, and has no voice repeaters coordinated within probably 200

miles, so no data on co-channel there.  The repeater is running about 15

watts into an inverted j-pole (hopeing for some downtilt).  The

interference problems we've seeen have been tracked down to the following

causes:

1) When the repeater was operated at 1200 baud by its previous owner, we

found that some handheld radios being used to talk to the repeater had their

PLL lockup time disimproved by the immediate application of packet audio on

keyup.  This caused the PLL to wander around for an extra 100 mS or so near

(but not on) channel.  This would kerchunk the adjacent channel repeaters.

In our case, the radio primarily responsible was the Icom IC-2AT.

2) The most recent case was caused by a station that did not have his

deviation set correctly, and due to extreme overdeviation, was bringing up

the the 146.96 machine.  147.00 was not affected particularly because it has

a slow squelch - added when the first problem appeared.

To address an area of specific concern:  When a station is set up for 9600

baud or faster packet operations, it is necessary to go directly to the

radio's modulator  as the modulation poin.  This bypasses the deviation

limiter and low pass filter.  It is critical to the sucessful operation of a

9600 baud system that the owners be made aware of their responsbility to

adjacent channel systems that they make sure all users have a way to get

their deviation set correctly (at 2.75 kHz for 9600).  Any more, and they'll

be un-neighborly - and as a rule the user's station won't work through the

repeater worth a darn anyway!

The other systems have not had any operating problems I know of, as far as

neighbors, etc.

Here is a little more grist for the mill:  Since packet repeaters are not

normally used by mobile stations, they do not normally need high gain, high

ERP, or to be able to service users on omni antennas.

Packet (and 9600 packet especially) is much more multi-path sensitive than

voice communications.  In this area, a beam is manditory for using the

repeaters, just to keep the multipath down.  So all the users use beams, and

the repeater uses a low gain (6 db or so) and low power (10 watts out of the

dupes) to acheive coverage that is appropriate for the level of multipath

that can be tolerated.

The advantage we plan to start excercising soon is that of re-using the data

repeater pairs, at distances where one site can be heard from another.  If

this sounds like a cellular approach, the fact that the 440 repeaters are at

cellular phone sites might be a hint :)

Because the users are using beams, we expect to be able to do this at 40-50

miles seperation on 440 MHz, initially.  This means that if you can find 3-6

pairs for 9600 baud 440 repeaters, you probably will not ever have to find

another - just arrange careful re-use.  Since no (sane) person sits around

and listens to packet, only the TNC's care, and with a good signal margin,

there should be no problem with the local machine capturing the receiver.

I think one of the reasons we've had good success in our coordination and

operational efforts with packet repeaters is that a number of the packet

folks are, or have been, involved in voice repeater coordination in the

past, and have learned the issues involved.  So if you have an interested

person or group, get them involved in the coordination process.  It will

give them a better understanding of what you are trying to accomplish, both

for them, and for the rest of the repeater community, and not leave them

flat-footed and frustrated when you bring up yet another technical issue or

question that they need to address.

Hope that helps.  73, Bob

----------------------------------------------------------------------------

|  Bob Donnell, KD7NM     bob@ethanac.kd7nm.ampr.org   rdonnell@eskimo.com |

|  Western Washington Amateur IP Address Coordinator   (206) 775-3651      |

----------------------------------------------------------------------------

> Hi Rick - you should contact the Texas VHF/FM Society.  We have a bandplan

> in place for digial repeaters.

>

> In addition, digital repeaters in the voice repeater are handled the same

> way -- except that they make sure they don't fudge on the repeater spacing.

> The main reason for this is that since packet operates nearly

> continously....a band opening can really caus problems.  The VHF/FM Society

> treis to get voice repeaters to operate with Toned Open-Sql.  In this way

> the repeater can leave there sql fairly tight and those on the firnges use

> tones to get the sql to open fully for them to make it.

>

> Greg

>

Additionally Rick, encouraging the voice systems that are co-channeled to

generate or pass CTCSS on the output allows those with CTCSS decoders to set

their radios to ignore non-CTCSS (packet) signals.  If the voice repeaters

are fitted with a demodulator and a 'true data-carrier detect' (True-DCD)

circuit that will >close< the squelch, then even a great band-opening might

be prevented from affecting the voice repeaters' users significantly.

Also, suggest (or make a condition of coordination) that the packet repeater

operators use True-DCD on the repeater, and then only valid data will bring

up the packet repeater.  Regenerating the data is also a good engineering

decision, rather than just passing through audio.  This applies to 1200 baud

systems.  The above are 'requirements' to a repeater at 9600.

73, Bob

----------------------------------------------------------------------------

|  Bob Donnell, KD7NM     bob@ethanac.kd7nm.ampr.org   rdonnell@eskimo.com |

|  Western Washington Amateur IP Address Coordinator   (206) 775-3651      |

----------------------------------------------------------------------------

Coordinating packet repeaters is different in Michigan than what it

seems to be like in other states.  The repeater council here didn't

want to do any coordination in digital modes, so a packet council

was set up for this. The 'MIchigan PAket Council' (MIPAC) was formed

and it handles all digital activities and coordination.  If we need

a packet repeater then MIPAC goes to the repeater council and just

requests the frequencies. The council really doesn't care what's on

the frequency, since if it's digital it becomes the responsibility

of MIPAC.   When MIPAC was formed and became incorporated the

repeater council recognized it as the digital coordinating body.

I know of 2 packet repeater pairs in Michigan off the top of my head.

One is a 1200 baud repeater and the other is 9600. Both are just outside

of the voice repeater bandplan. We currently don't share the frequencies

with voice repeaters and there really isn't a need to around here either.

Plus there just aren't any voice pairs available. We ran out of 2m, 220

and 440 pairs years ago, so if anyone wants a repeater they have to

either get a snapper pair (limited power, range, PL, odd offset, etc...)

or go digital and move outside of the voice range. Of course the lack

of frequencies is in the SouthEast Michigan area (around Detroit). Can't

speak on the rest of the state, but MIPAC does cover all of it.

Just in case anyone from the Upper pennisula of Michigan is on here, to

prevent offending them  :-)    Nope the Michigan Repeater Council doesn't

cover the U.P.  I think Wisconsin handles Upper Michigan.

--

Ron N8FOW

AMPRnet  : n8fow@n8fow.ampr.org

Internet : ron@chaos.eng.wayne.edu

aa011@detroit.freenet.or

---------------------------------------------------------------------------------

In article <9410242234.AA11994@ATK.COM> you wrote:

: I ask because it appears that the reported successful mingling of 9.6 Kbd

: digital facilities with voice repeaters on 2M were in geographical areas (or

: sub-bands) with 20 KHz channels.  In this part of the country the 2M repeater

: coordination plan uses 15 Khz channels above 145 MHz.

We also (Colorado) have the brain-damaged 15 khz channel spacing on 2m, but we don't have a 9600 baud repeater on 2m, so I haven't worried about it.

Not interested in building one, either.  70cm is so much more pleasant to work

with... the cans are rack-mountable, antennas have lower wind load for a given amount of gain, etc...

73 - Bdale, N3EUA

======================================================================================

[Internet 1-4-95]

Date: 02 JAN 95 21:51:10 EST

From: "Harvey, Nels" <NHAR@MUSIC.LIB.MATC.EDU>

Subject: Packet Repeater Coordination

I have noted some discussion of the use of Voice Repeater frequencies

for Packet repeaters.  As Vice President of the Mid-America

Coordination Council (MACC), I would like to point out that we have

given reasonable consideration of the need for Packet repeaters, to

provide efficient throughput.  MACC is the umbrella coordination group

for over 20 state and local repeater coordination groups.

In essence, it has been determined that Packet and voice repeaters are

generally not compatible.  MACC has a basic bandplan providing for

Packet from 446.025 MHz. to 446.200 MHz. simplex, and duplex pairs of

441.025/446.025, 441.050/446.050, and 441.075/ 446.075 MHz.  These are

chosen to provide frequencies in the 440 MHz. range to Packet interests

without a need for coordination with the voice repeater coordinators,

and to protect unpublished repeater control and audio links.  These

frequencies, along with the 430 MHz. frequencies that are harder to get

some radios to work on, provide the Packet users with a reasonable

number of frequencies for most Packet applications, I believe.

The point is, we do wish to work closely with Packet organizations,

because we are using the same limited resourse.  I believe our plan is

reasonable, easy to use, and places voice repeaters at a minimum risk.

If further frequencies are needed, discussions with MACC are welcomed.

Our Annual Meeting is at the Dayton Hamvention each year.  The members

are in the Repeater Directory.

Coordination is a way of life in Amateur Radio.  We don't really need

any range wars like the ones in the 70's any more.  MACC, TMARC, SERA,

TSARC and others are umbrella organizations that offer support and a

measure of authority for our Coordination Warriors.  We all need to

hear from the Packet community with your frequency needs.  I hate those

re-tries as much as anyone else.  Together, we can minimize

interference, and further this great hobby to greater heights!

73, Nels....

Nels Harvey, WA9JOB    Chairman, Wisconsin Association of Repeaters

Television Engineer            E-Mail:  NHAR@MUSIC.LIB.MATC.EDU

WMVS/WMVT Milwaukee            Packet: WA9JOB@WA9POV.EN63FE.WI.USA.NOAM

Milwaukee Area Technical College       Work Phone:  (414) 271-1036

Milwaukee, WI 53233-1443               Fax:         (414) 297-7536

Home Phone:  (414) 241-3769

------------------------------

Date: Tue, 3 Jan 1995 13:27:30 GMT

From: gfiber@halcyon.com (Gary P. Fiber)

Subject: Packet Repeater Coordination

In article <02JAN95.23601075.0023@ESAMATC.LIB.MATC.EDU> "Harvey, Nels" <NHAR@MUSIC.LIB.MATC.EDU> writes:

>From: "Harvey, Nels" <NHAR@MUSIC.LIB.MATC.EDU>

>Subject: Packet Repeater Coordination

>Date: 02 JAN 95 21:51:10 EST

[Nels message (see above) deleted]

Nels,

Here in Seattle, WA we have a 9600 bps repeater at 146.980 MHz and a voice

repeater at 146.960 MHz.  Never have any interference problems here.  I think

just good sound technical practice will allow the two to co-exhist.   Also we

have 3 -440 MHz 9600 bps packet repeaters too, all are within the " voice "

repeater freqquency range.  We are right next to some voice repeaters there

too and no trouble.  Now the deviation is set to 3 KHz, not 5 or 7 but 3 KHz

and all works quite nicely.

73' Gary Fiber

-----------------------------------------------------------------------